�



3ComCourier™ �I-modem™







New Features

We have added the following new features to your I-modem:

V.90 (56K standard)

Selective Reject

Disabling reset to view previous call data 

Fast Answer

Turning off the echo canceller (2100hz tone)

Also included in this addendum are all AT commands needed to use new features and new Connect messages.

V.90

The I-modem now has V.90,  the ITU 56K standard.  The upstream and downstream channels can operate at the following symbol rates: 3000, 3200, 3429.  The V.34, 2400, 2743 and 2800 symbol rates are no longer supported.  

In V.90 mode, the negotiation of symbol rates is done automatically. However, individual symbol rates can be disabled.  To disable individual rates use S-Register 54.  See Appendix A, Alphabetic Command Summary, in the Command Reference for more information.

Selective Reject

The I-modem now supports Selective Reject for analog calls. Selective Reject improves performance on noisy lines by reducing the amount of overhead incurred when the protocol must resend data due to errors.

When Selective Reject is active, only the frame that contained the error is resent, instead of the frame plus all of the following unacknowledged frames.

Selective Reject is an optional part of the ITU-T V.42 (LAPM) standard. 

By default, Selective Reject is enabled.

Disabling reset to view previous call data

If the communication software you use with your I-modem cannot be configured to stop sending the ATZ string before or after each connection, set your I-modem to  ignore ATZ.  When the I-modem ignores ATZ, it does not clear out the previous call statistics in the ATI6 screen.

By default, this feature is disabled.

Fast Answer

Fast Answer saves time by forcing the I-modem to answer calls as fast as possible without going through the Ring Indicate toggling sequence.

By default, Fast Answer is disabled.

Turning off the echo canceller (2100hz tone)

You can now configure the I-modem so it does not send a 2100hz tone at the beginning of a call.

When Data Over Voice (DOV) is enabled, the I-modem sends a 2100hz tone at the beginning of a 3.1khz 56 kbps voice call to turn off the telephone company’s echo canceller.

If there are no echo cancellers on the B-channel during the call, disable the 2100hz tone in DOV to save 2 seconds of connect time.

Note: When you disable the tone, you also risk not being able to connect to the phone number.

By default, this feature is enabled; the I-modem sends a 2100hz tone at the beginning of a DOV call.

New AT Commands

To do this�Command�Command��Enable V.90 (Default)�ATS58.5=0���Disable V.90�ATS58.5=1�ATS58=32��Disable 2100hz tone in DOV�ATS69.5=1�ATS69=64��Disable External Plug and Play�ATS69.0=1�ATS69=1��Disable ML-PPP tones�ATS69.1=1�ATS69=2��Disable Selective Reject�ATS51.6=1�ATS51=2��Enable Data Over Voice�ATS69.6=1�ATS69=32��Enable Fast Answer�ATS69.7=1�ATS69=128��Ignore ATZ�ATS69.2=1�ATS69=4��Note: S69.3 and S69.4 are reserved.

New Connect Messages

Message �#��28000�256��28000/ARQ�257��28000/V90�258��28000/ARQ/V90�259��29333�260��29333/ARQ�261��29333/V90�262��29333/ARQ/V90�263��30666�264��30666/ARQ�265��30666/V90�266��30666/ARQ/V90�267��32000�268��32000/ARQ�269��32000/V90�270��32000/ARQ/V90�271��34666�272��34666/ARQ�273��34666/V90�274��34666/ARQ/V90�275��36000�276��36000/ARQ�277��36000/V90�278��36000/ARQ/V90�279��38666�280��38666/ARQ�281��38666/V90�282��38666/ARQ/V90�283��40000�284��40000/ARQ�285��40000/V90�286��40000/ARQ/V90�287��33333/V90�288��33333/ARQ/V90�289��37333/V90�290��37333/ARQ/V90�291��41333/V90�292��41333/ARQ/V90�293��42666/V90�294��42666/ARQ/V90�295��44000/V90�296��44000/ARQ/V90�297��45333/V90�298��45333/ARQ/V90�299��46666/V90�300��46666/ARQ/V90�301��48000/V90�302��48000/ARQ/V90�303��49333/V90�304��49333/ARQ/V90�305��50666/V90�306��50666/ARQ/V90�307��52000/V90�308��52000/ARQ/V90�309��53333/V90�310��53333/ARQ/V90�311��54666/V90�312��54666/ARQ/V90�313��56000/V90�314��56000/ARQ/V90�315��57333/V90�316��57333/ARQ/V90�317��58666�318��58666/ARQ�319��58666/V90�320��58666/ARQ/V90�321��60000�322��60000/ARQ�323��60000/V90�324��60000/ARQ/V90�325��61333�326��61333/ARQ�327��61333/V90�328��61333/ARQ/V90�329��62666�330��62666/ARQ�331��62666/V90�332��62666/ARQ/V90�333��64000/V90�334��64000/ARQ/V90�335��

1.024.1362-00










