Courier I-modem
Release Notes

New Features

The following new features have been added to your modem since the 2.2.2 release:

- V.90 Server Capabilities

- Microsoft Encrypted Password Authentication

- Outgoing V.110 Rate Selection

- A-Law and u-Law Companding Methods

Year 2000 Compliance

For information on Year 2000 compliance and 3Com products, visit the 3Com Year 2000 web page:

http://www.3com.com/products/yr2000.html

New AT Commands

The following table includes a list of all AT commands needed to use these new features. See the Command Reference for more information regarding bit-mapped registers.

| In order to |  |  |  | Command |  |
| --- | --- | --- | --- | --- | --- |
| Enable V.90 (Default) |  |  |  | ATS58.5=0 |  |
| Disable V.90 |  |  |  | ATS58.5=1 |  |
| Disable the asterisk (*) key on the analogue device port as a redial |  |  |  | ATS68.5=1 |  |
| Use the asterisk (*) key as redial |  |  |  | ATS68.5=0 |  |
| Enable V.110 fixed rate calls (38.4 kbps) |  |  |  |  | ATS80.3=1 |
| Use S-Register 46 to set the outgoing V.110 rate. |  |  |  |  | ATS80.3=0 |
| S46= | kbps |  |  |
| 0 | 38.4 |  | ATS46=0 |
| 1 | 19.2 |  | ATS46=1 |
| 2 | 9.6 |  | ATS46=2 |
| 3 | 4.8 |  | ATS46=3 |
| 4 | 2.4 |  | ATS46=4 |
| 5 | 1.2 |  | ATS46=5 |
| 6 | 0.6 |  | ATS46=6 |
| Enable A-Law encoding See the Command Reference Manual for more information regarding the switch protocol type. |  |  |  |  | AT*W=nA where n is the switch protocol type. |
| Enable u-Law encoding See the Command Reference Manual for more information regarding the switch protocol type. |  |  |  |  | AT*W=nU where n is the switch protocol type. |

V.90

The I-Modem now has V.90 server capabilities. By default, V.90 server is enabled on your modem.

V.90 analogue clients can call into your I-Modem to get V.90 connections. The analogue device port is not able to act as a V.90 server.

Microsoft Encrypted Password

If the I-Modem is connecting to a server that supports Microsoft Encrypted Password Authentication, the I-Modem will respond to the server using the same protocol. However, the I-Modem will communicate with your computer using Password Authentication Protocol (PAP).

Since your computer must communicate using PAP, the software on your computer should not be configured to ONLY accept Microsoft Encrypted Password.

Your modem will translate PAP into Microsoft Encryption Password Authentication to communicate with the server. The password will be clear-text over the serial cable between your computer and the modem. Once the password is on the ISDN telephone network, the password will be encrypted.

Outgoing V.110 Rate Selection

You can now select the outgoing V.110 rate. This feature was added to your I-Modem for compatibility. It allows each user to set a specific default outgoing rate.

A-Law and u-Law Companding Methods

You can choose companding methods for a particular switch for the analogue device port and V.90 calls. Companding methods will either be A-Law or u-Law. To set the companding method to A-Law send AT*W=nA (where n is the switch protocol type) to the I-Modem. See the Command Reference Manual for more information about the switch protocol type.

New Connect Messages

The table below includes all new connect messages.

| Message | # | Message | # |
| --- | --- | --- | --- |
| 300/DIGITAL | 244 | 28000/ARQ/V90 | 259 |
| 600/DIGITAL | 245 | 29333 | 260 |
| 1200/DIGITAL | 246 | 29333/ARQ | 261 |
| 2400/DIGITAL | 247 | 29333/V90 | 262 |
| 3600/DIGITAL | 248 | 29333/ARQ/V90 | 263 |
| 4800/DIGITAL | 249 | 30666 | 264 |
| 7200/DIGITAL | 250 | 30666/ARQ | 265 |
| 9600/DIGITAL | 251 | 30666/V90 | 266 |
| 12000/DIGITAL | 252 | 30666/ARQ/V90 | 267 |
| 14400/DIGITAL | 253 | 32000 | 268 |
| 19200/DIGITAL | 254 | 32000/ARQ | 269 |
| 38400/DIGITAL | 255 | 32000/V90 | 270 |
| 28000 | 256 | 32000/ARQ/V90 | 271 |
| 28000/ARQ | 257 | 34666 | 272 |
| 28000/V90 | 258 | 34666/ARQ | 273 |
| Message | # | Message | # |
| 34666/V90 | 274 | 44000/V90 | 296 |
| 34666/ARQ/V90 | 275 | 44000/ARQ/V90 | 297 |
| 36000 | 276 | 45333/V90 | 298 |
| 36000/ARQ | 277 | 45333/ARQ/V90 | 299 |
| 36000/V90 | 278 | 46666/V90 | 300 |
| 36000/ARQ/V90 | 279 | 46666/ARQ/V90 | 301 |
| 38666 | 280 | 48000/V90 | 302 |
| 38666/ARQ | 281 | 48000/ARQ/V90 | 303 |
| 38666/V90 | 282 | 49333/V90 | 304 |
| 38666/ARQ/V90 | 283 | 49333/ARQ/V90 | 305 |
| 40000 | 284 | 50666/V90 | 306 |
| 40000/ARQ | 285 | 50666/ARQ/V90 | 307 |
| 40000/V90 | 286 | 52000/V90 | 308 |
| 40000/ARQ/V90 | 287 | 52000/ARQ/V90 | 309 |
| 33333/V90 | 288 | 53333/V90 | 310 |
| 33333/ARQ/V90 | 289 | 53333/ARQ/V90 | 311 |
| 37333/V90 | 290 | 54666/V90 | 312 |
| 37333/ARQ/V90 | 291 | 54666/ARQ/V90 | 313 |
| 41333/V90 | 292 | 56000/V90 | 314 |
| 41333/ARQ/V90 | 293 | 56000/ARQ/V90 | 315 |
| 42666/V90 | 294 | 57333/V90 | 316 |
| 42666/ARQ/V90 | 295 | 57333/ARQ/V90 | 317 |

Copyright © 1998, 3Com Corporation. All rights reserved.
Unless otherwise indicated, 3Com registered trademarks are registered in the United States and may or may not be registered in other countries.

3Com, Boundary Routing, LANplex, LinkBuilder, NETBuilder, NETBuilder II, Parallel Tasking, EtherDisk, EtherLink, EtherLink II, NetAge, SmartAgent, SuperStack, TokenDisk, TokenLink, Transcend, and ViewBuilder are registered trademarks of 3Com Corporation. FDDILink, FMS, and NetProbe are trademarks of 3Com Corporation. 3ComFacts is a service mark of 3Com Corporation

Other brand and product names may be registered trademarks or trademarks of their respective holders.