# After a Successful Upgrade

## Windows 3.x users

Congratulations! You can now download at x2™ speeds with:

- Leading Internet service providers. For a list of the providers currently offering x2 connections, check our web page at http://x2.usr.com/connectnow

- Our BBS at 847-982-5092

Note: The first time you use your communications software program after the 56K upgrade, you have to tell the software program that you’ve upgraded. (In the RapidComm program, open the Setup menu, click Port and click Autodetect.)

## Windows 95/Windows NT 4.0  users

1  Double-click My Computer on your desktop

2  Double-click Control Panel.

3  Double-click Modem.

4  When you see this screen, highlight  the old modem’s description (the modem that you have upgraded). To highlight the description, click it. Then click Remove.

5  When the description has disappeared, click Add.

6  When you see this screen, select Don’t detect my modem….
Then click Next.

7  You will see a series of screens as the driver database builds.

8  When you see this screen:
1) Insert the Connections CD from this kit into your CD-ROM drive.
2) Click Have Disk.

9  When you see this screen, click the down arrow to select the letter for your CD-ROM drive.

Note: If your CD-ROM drive is assigned a letter other than “D,” select that letter instead.

Then click OK.

10  When you see this screen:
ùUnder Manufacturers, click U.S. Robotics Access Corp.
ùUnder Models, click the description of your upgraded modem.

11 When you see this screen, click the Communications Port your modem uses. If you forget the number, refer to page 5, where you wrote it down.

Note: In your system the Sportster might be installed on a different COM port than the one shown above.

Then click Next.

12  Windows will install your upgraded modem.

When you see a screen that says Your modem has been set up successfully, click Finish.

13  You’ll see the “Modem Properties” screen again. A description of your upgraded modem should appear in the field.


Click Close.

Congratulations! You can now download at x2™ speeds with:

- Leading Internet service providers. For a list of the providers currently offering x2 connections, check our web page at http://x2.usr.com/connectnow

- Our BBS at 847 982 5092

Note: The first time you use your communications software program after the 56K upgrade, you have to tell the software program that you’ve upgraded. (In the RapidComm program, on the Setup menu, click Modem, and click the down arrow to select the best description for your new modem.)

## Macintosh users

Congratulations! You can now download at x2™ speeds with:

- Leading Internet service providers. For a list of the providers currently offering x2 connections, check our web page at http://x2.usr.com/connectnow

- Our BBS at 847-982-5092