# Enhanced x2 Features

Your V.Everything with x2 has new result codes and the following new features.

| To do this | Use this command |
| --- | --- |
| Detemine if x2 is enabled in your modem | ATI7 |
| Limit the upper speed limit of an x2 connection | AT&N |
| Limit the lower speed limit of an x2 connection | AT&U |

Note: New x2 features should be transparent to most users. If you are an advanced user see Appendix B, “Controlling Link Speeds with &N and &U,” for detailed information regarding these new features.

## Controlling x2

Use the following S58 settings to control x2:

| To do this | Use this command |
| --- | --- |
| Disable x2 | ATS58.0=1 |
| Force A-law mode | ATS58.2=1 |

Note: A-law is required in all countries but the United States, Canada, Japan, Taiwan, and Hong Kong. If you are using your V.Everything in one of these countries, do not force A-law mode.

## Configuring the High Speed (HS) LED

You can configure your V.Everything to alert you when it reaches x2 speeds. Use the following S69 setting to configure the HS (High Speed) LED:

| To do this | Use this command |
| --- | --- |
| Configure the HS (High Speed) LED to turn red when your modem reaches speeds over 33.3 kbps. | ATS69=12 |