# Troubleshooting

## Troubleshooting x2 Client Connections

Use the chart below to understand issues affecting your V.Everything modem and how to fix them:

| Step | This may be the issue | Do this |  |
| --- | --- | --- | --- |
| 1 | x2 may not be enabled on your V.Everything. | See the section “How to Tell if x2 is Enabled” (Use the ATI7 command) |  |
| 2 | Several conditions may exist. | Use the ATI11 command and check the “x2 status” field for more information. |  |
|  |  | If this message appears | You cannot use x2 because |
|  |  | “Multiple CODECS in channel” | There are multiple analog-to-digital conversions on the channel. |
|  |  | “Remote modem is not x2” | The remote modem does not support x2. |
|  |  | “Channel is x2-capable but feature is not installed” | You have not purchased x2. |
| 3 | Your V.Everything may be connected to the public network via a PBX or other telephone equipment with analog-to-digital and digital-to-analog conversions. | Contact your telephone equipment vendor for information about obtaining pure analog service. Due to extra analog-to-digital conversions performed by some PBX’s and other telephone equipment, x2 client modems may not be able to make x2 connections. |  |
| 4 | There is another issue. | Do the following: - Visit the U.S. Robotics x2 Web Site at http://www.usr.com/x2 - Contact U.S. Robotics Technical Support at 800.550.7800 |  |

### During the upgrade, if something happens that you cannot work through, call:

- For V.Everything products: 800-231-8770

### After the upgrade, if you have questions about your modem’s performance or how to operate it:

See your modem’s manual.