# Controlling Link Speeds with &N and &U

You can use the &N and &U commands to control link speeds. Couriers without x2 can still use the &N and &U commands, but can only control link speeds up to 33.6 kbps.

## Controlling Link Speeds

You can use the &N and &U commands to control the link speeds of your Courier V.Everything with x2. Use the following table to determine how to use &N and &U commands:

| To limit the | Use |
| --- | --- |
| Highest possible connect speed | &N |
| Lowest possible connect speed | &U |
| Range of possible connect speeds | &N and &U |

Note: The default values for &N and &U are 0. If you change these values, you will limit the speeds at which you can connect. U.S. Robotics recommends that you do not alter these values.

## Limiting the Highest Possible Connect Speed

The &N command allows you to limit the highest possible connect speed. If a remote modem attempts to connect to your Courier with x2 at a speed higher than &N, your Courier with x2 will not allow it to connect.

| To limit the | Use this command | Where x is |
| --- | --- | --- |
| Highest possible connect speed | AT&N=x | A value from 0 to 32 |

Note: See table B-1 for a list of connect speed values.

## Limiting the Lowest Possible Connect Speed

The &U command allows you to limit the lowest possible connect speed. If a remote modem attempts to connect to your Courier with x2 at a speed lower than &U, your Courier with x2 will not allow it to connect.

| To limit the | Use this command | Where x is |
| --- | --- | --- |
| Lowest possible connect speed | AT&U=x | A value from 0 to 32 |

Note: See table B-1 for a list of connect speed values.

## Limiting a Range of Possible Connect Speeds

By setting &N and &U values, you can limit the range of speeds at which your Courier with x2 connects. If a remote modem does not connect to your Courier with x2 at a range between the speeds designated by the &N and &U commands, your Courier with x2 will not allow it to connect.

Note: The link speed associated with the &U argument cannot be greater than the link speed associated with &N argument.

## &N and &U Command Values

Use the following table for a complete list of &N and &U link speeds and their associated indexes:

| Link Speed | Index |  | Link Speed | Index |  | Link Speed | Index |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Highest | 0 |  | 21600 | 11 |  | 45333 | 22 |
| 300 | 1 |  | 24000 | 12 |  | 46666 | 23 |
| 1200 | 2 |  | 26400 | 13 |  | 48000 | 24 |
| 2400 | 3 |  | 28800 | 14 |  | 49333 | 25 |
| 4800 | 4 |  | 31200 | 15 |  | 50666 | 26 |
| 7200 | 5 |  | 33600 | 16 |  | 52000 | 27 |
| 9600 | 6 |  | 33333 | 17 |  | 53333 | 28 |
| 12000 | 7 |  | 37333 | 18 |  | 54666 | 29 |
| 14400 | 8 |  | 41333 | 19 |  | 56000 | 30 |
| 16800 | 9 |  | 42666 | 20 |  | 57333 | 31 |
| 19200 | 10 |  | 44000 | 21 |  | 64000 | 32 |

Table B-1  Link Speeds and Indexes

Note: For x2-mode links, &N and &U are used to constrain the speed of the higher speed direction of the link.