Sportster Development Kit for Windowsä

Version  2.0

March 12, 1996

*

Disclaimer

The material in this manual if for information purposes and is subject to change without notice. U.S. Robotics makes no representations or warranties with respect to the contents hereof and specifically disclaims any implied warranties of MERCHANTABILITY or fitness for any particular purpose.

TABLE OF CONTENTS

1. INTRODUCTION

1.1 SDK Support and How To Get The SDK

1.2 Applicable Reference Documents

1.3 Acronyms and Definitions

1.4 Trademarks and Copyrights

1.5 Diagramming Symbols

2. ARCHITECTURE

2.1 API Support

2.2 Windows 95 Dial-Up Networking

3. TAPI  SUPPORT

3.1 Example Of An Outbound Call and Data Passing

3.2 Line Devices

3.3 Address Capabilities

3.4 Device Capabilities

3.5 Call Information

3.6 Phone Capabilities

3.7 Data Device

3.8 The Voice Device

3.9 Events Generated

3.10 Device Specific Extensions

4. STREAMS LIBRARY

4.1 IscOpenStream

4.2 IscCloseStream

4.3 IscWriteStream

4.4 IscReadStream

4.5 IscStreamLookAhead

4.6 Invalidating A Data Handle

5. WINISDN

1. - Telephony Functional Level Guidelines

REVISION HISTORY

Description of Change					Revision	Date

Created Specification					 1.0	      	Sept. 12, 1995

Final Release of first draft.				               1.0		Nov. 20, 1995

Added list server for support.

Changes to SDK for new products and			2.0		March 12, 1996

for easier support of multiple versions in the

future.

# INTRODUCTION

This document presents a general  overview of the features supported by the  Sportster ISDN 128K for Windows 95 and Windows 3.x Internet Access.  It  describes the interfaces supported that applications developers can access  for developing applications that take advantage of the Sportsters capabilities.

## SDK Support and How To Get The SDK

The Sportster SDK will be supported through an automated mailing list program called Majordomo. To get access to the list you should use the following addresses.

List Posting Address:  sp128sdk@elroy.usr.com

List Request Address: sp128sdk-request@elroy.usr.com

To submit something to the Sportster SDK  list, you should simply mail it to the list posting address.

To get information on how to use the list and to subscribe you should send your request

to  the List Request address shown above. If you include the command help in the message body then the Majordomo will return a list of commands you can use and what they can do.

To be added to the list, you  simply send a message of the form

subscribe sp128sdk

by email to  majordomo@elroy.usr.com.

Majordomo understands several commands, and is not limited to a

single command per message (it will process commands until reaching

end-of-message or the command "end").  The command "help" will tell

you about all the other commands.

To retrieve the latest copy of the Sportster SDK you will need to send the get command to the

list  server for the file sdk.zip. This will be a pkzip archive that  will contain our stream library,

header files, samples and so forth. This file should be unzipped using the command pkunzip -o -d sdk.

To get access to the telephony SDK, which will be needed if you plan on developing to the TAPI interface, you can obtain the SDK from Microsoft or look in the /developr/tapi directory on the Microsoft FTP site or do a keyword search on TAPI in the WINEXT forum of CompuServe.

## Applicable Reference Documents

[TAPI10] “Telephony Applications Programming Interface” Specification,  Version 1.0, dated  1993.

[TSPI10]  “Telephony Service Providers  Interface” Specification, Version 1.0, dated  1993 .

[WINISDN] “WinISDN An Open Specification for ISDN voice and data communications“, dated 2 February 1995.

## Acronyms and Definitions

NDIS	Network Driver Interface Specification.

TAPI	Telephony Application Programming Interface.  The interface implemented by applications which allows them to  perform telephony operations on a hardware device in a seamless fashion.

TSPI	Telephony Service Provider Interface. This is the interface supplied by hardware  vendors for allowing TAPI based applications to access their device.

WinISDN  Specification developed by ISDN*Tek and NetManage for communicating with ISDN  cards in the Windows environment.

## Trademarks and Copyrights

Windows is a registered trademark of Microsoft Corporation.

Sportster  is a registered trademark of  U.S. Robotics.

## Diagramming Symbols

The LEGEND below shows the symbols which are used throughout this document  for the various diagrams.

# ARCHITECTURE

The overall architecture for the Sportster Adapter is shown in Figure 1 - Functional Architecture for the Sportster Driver Set.  As can be seen, the drivers are sub-divided into two areas: Protocol Drivers and Core Drivers. Core Drivers are responsible for providing access to the adapter, ISDN support and call control. Protocol drivers support  various features which  allow the applications access to the adapter for remote LAN access, dial-up networking, telephony and so forth. Protocol drivers are accessed either through an API that is supported by the Sportster  or they fit into a Windows protocol stack and are therefore transparent to the applications above them.

Figure 1 - Functional Architecture for the Sportster Driver Set.

## API Support

The Sportster supports two APIs at the Windows level that applications can directly use. These APIs are TAPI and WinISDN.  The WinISDN specification has built in support for call control and data passing. The TAPI specification supports call control only. Data passing when using this API is done with the Sportster  Streams library.

Figure 2 -  Windows APIs for the Sportster

## Windows 95 Dial-Up Networking

Dial-up networking  for Windows 95 is provided by the Sportster  Dial-up networking driver. This driver resides within the WAN miniport architecture.

# TAPI  SUPPORT

This section  presents  an overview of the TAPI support offered by the Sportster. All call control down to the Sportster ISDN driver  is performed using TAPI. Through the use of TAPI, the application gains access to first party call control for both voice and data calls.  In addition to this, several very useful device specific extensions have been added through TAPI which allow applications to configure the Adapter, start and re-start the Adapter, obtain statistics and so forth.

## Example Of An Outbound Call and Data Passing

Once an application has established a call, or received one, it  then has access to the data stream for that call by using the Streams Library. To gain access to the data stream, the application would  need to call lineGetID() for the “isc/stream” to obtain the Stream Identifier. Once this has been done, the application is free to send and receive data for the call. This data stream is only valid for as long as the call is active.

The  typical sequence that an application would go through to make an outbound digital data call and pass data is shown in Figure 3 - Calling sequence for  an outbound digital data call.

Figure 3 - Calling sequence for  an outbound digital data call.

As can be seen from the figure, the application would first open a Sportster data line device. By opening the device, the application gains access to the Adapter for call control and establishes an event callback so that it can receive notification of the individual call states. Once the line device has been opened, the application can choose either to wait for an incoming call or initiate a call. To initiate a call, the application would use the lineMakeCall() function.  Once established, the application can then get a data handle for the call using the lineDevSpecific() function call and register its callback functions. If successful, the application can then send and receive data.

The Sportster  provides TAPI support for Plain Old Telephone Service (POTS) and digital data calls.  The POTS supports interactive voice using an analog telephone off the RJ-11 jack on the card. Although compatible devices may be used, e.g. a telephone answering machine, the capability to control and manage them through a TAPI application is not supported.

From a TAPI perspective,  each  Sportster  card is viewed as having three line devices which share the same address (phone number) and draw from a common pool of 64Kbps B channels. The determination of the B channel to use for voice or data is dynamic and is not programmable through an application.  If a B channel is actively being used for voice, the additional  channel can be used for data transfer, simultaneous to a Windows TAPI client using the voice channel. If both B channels are being used for data transfer then one channel can be brought down for a voice call.

## Line Devices

The Sportster  provides three line devices and one phone device for each board. There is one line device for Interactive Voice and two for Digital Data.  These devices are further discussed in the following sections.

## Address Capabilities

The address capabilities (phone numbers) for the Sportster report the following. Any field not shown is usually set to zero and therefore not supported.

Address Caps Fields			Value(s)

dwAddressSharing		BRIDGED_NEW

dwAddressStates			LINEADDRESSSTATE_INUSEZERO,

LINEADDRESSSTATE_INUSEONE,

LINEADDRESSSTATE_INUSEMANY ,

LINEADDRESSSTATE_OTHER ,

LINEADDRESSSTATE_NUMCALLS

dwCallInfoStates 		LINECALLINFOSTATE_OTHER,

LINECALLINFOSTATE_APPSPECIFIC,

LINECALLINFOSTATE_RATE,

LINECALLINFOSTATE_REASON,

LINECALLINFOSTATE_TRUNK,

LINECALLINFOSTATE_CALLID,

LINECALLINFOSTATE_RELATEDCALLID,

LINECALLINFOSTATE_MONITORMODES,

LINECALLINFOSTATE_ORIGIN,

LINECALLINFOSTATE_MEDIAMODE,

LINECALLINFOSTATE_CALLERID,

LINECALLINFOSTATE_CALLEDID,

LINECALLINFOSTATE_DIALPARAMS

dwConnectedIDFlags

dwRedirectionIDFlags

dwRedirectingIDFlags

dwCallerIDFlags

dwCalledIDFlags			LINECALLPARTYID_ADDRESS,

LINECALLPARTYID_UNKNOWN,

LINECALLPARTYID_UNAVAIL

dwCallStates			LINECALLSTATE_IDLE,

LINECALLSTATE_OFFERING,

LINECALLSTATE_ACCEPTED,

LINECALLSTATE_DIALTONE,

LINECALLSTATE_DIALING,

LINECALLSTATE_RINGBACK ,

LNECALLSTATE_BUSY,

LINECALLSTATE_SPECIALINFO,

LINECALLSTATE_CONNECTED,

LNECALLSTATE_PROCEEDING,

LINECALLSTATE_DISCONNECTED,

LINECALLSTATE_UNKNOWN

dwDialToneModes 		LINEDIALTONEMODE_NORMAL,

LINEDIALTONEMODE_SPECIAL,

LINEDIALTONEMODE_UNKNOWN,

LINEDIALTONEMODE_UNAVAIL

dwBusyModes 			LINEBUSYMODE_STATION,

LINEBUSYMODE_TRUNK,

LINEBUSYMODE_UNKNOWN,

LINEBUSYMODE_UNAVAIL

dwSpecialInfo   			LINESPECIALINFO_NOCIRCUIT,

LINESPECIALINFO_CUSTIRREG,

LINESPECIALINFO_REORDER,

LINESPECIALINFO_UNKNOWN,

LINESPECIALINFO_UNAVAIL

dwDisconnectModes 		LINEDISCONNECTMODE_NORMAL ,

LINEDISCONNECTMODE_UNKNOWN,

LINEDISCONNECTMODE_REJECT ,

LINEDISCONNECTMODE_BUSY,

LINEDISCONNECTMODE_NOANSWER,

LINEDISCONNECTMODE_BADADDRESS ,

LINEDISCONNECTMODE_CONGESTION ,

LINEDISCONNECTMODE_INCOMPATIBLE,

LINEDISCONNECTMODE_UNAVAIL

dwAddrCapFlags       		LINEADDRCAPFLAGS_ORIGOFFHOOK,

LINEADDRCAPFLAGS_DIALED ,

LINEADDRCAPFLAGS_PARTIALDIAL (VOICE ONLY)

dwCallFeatures      		LINECALLFEATURE_DIAL,

LINECALLFEATURE_ACCEPT,

LINECALLFEATURE_SECURECALL ,

LINECALLFEATURE_ANSWER ,

LINECALLFEATURE_DROP

dwRemoveFromConfCaps		LINEREMOVEFROMCONF_NONE

dwRemoveFromConfState		LINECALLSTATE_IDLE;

dwMaxNumActiveCalls			Two for data lines and One for voice.

## Device Capabilities

The device capabilities  reported by the Sportster  line devices are listed below. The values are common for all line devices  unless noted.  Any fields not shown are set to zero and therefire probably not used.

Device Caps Fields			Value(s)

Provided there is room in the buffer, the following variable sized fields are filled in:

dwProviderInfoSize			Our driver name and version.

dwProviderInfoOffset

dwLineNameSize   			The name of the line device given by us.

dwLineNameOffset

dwSwitchInfoSize			Name of the switch user configured.

dwSwitchInfoOffset

dwSwitchInfoOffset

dwPermanentLineID			Our permanent device id.

dwStringFormat				STRINGFORMAT_ASCII,

STRINGFORMAT_BINARY

dwAddressModes			LINEADDRESSMODE_ADDRESSID,

LINEADDRESSMODE_DIALABLEADDR

dwNumAddresses			One or Two. Depends upon number of phone numbers

configured.

dwMediaModesSupported			For voice it is INTERACTIVEVOICE.

For data it is DIGITALDATA.

dwBearerModesSupported		For voice it is  VOICE.

For data it is  DATA.

dwMaxRateSupported			For voice it is 64Kbps.

For data it is 128Kbps.

dwRingModes				Nine including silent. These are pre-defined ring patterns

for the SecureLink board.

dwMaxNumActiveCalls  			Two (2).

dwAnswerMode 				LINEANSWERMODE_DROP;

dwLineStates         			LINEDEVSTATE_OTHER ,

LINEDEVSTATE_RINGING ,

LINEDEVSTATE_CONNECTED ,

LINEDEVSTATE_DISCONNECTED,

LINEDEVSTATE_OUTOFSERVICE,

LINEDEVSTATE_INSERVICE,

LINEDEVSTATE_OPEN,

LINEDEVSTATE_CLOSE,

LINEDEVSTATE_NUMCALLS,

LINEDEVSTATE_REINIT

dwGenerateToneModes  			LINETONEMODE_BUSY,

LINETONEMODE_RINGBACK,

LINETONEMODE_CUSTOM

dwGenerateToneMaxNumFreq		Zero (0)

dwGenerateDigitModes 			LINEDIGITMODE_DTMF

dwMonitorDigitModes  			LINEDIGITMODE_DTMF

dwDevSpecificSize			Set to device specific defaults for the line.

This will be an ISCCALLSPECIFIC structure.

dwDevSpecificOffset			Index into buffer of where this record starts.

## Call Information

The  call information returned by a GetCallInfo is described below.

CALLINFO Field		Description

dwLineDeviceID			The line device idenitifer.

dwAddressID			Address id for the call.

dwBearerMode			Data or Speech.

dwRate				Speed of the call in bits per second.

dwMediaMode			Digital Data or Voice.

dwAppSpecific			Returned if application set app specific data.

dwCallID			ISDN Call Appearance.

dwRelatedCallID			Not Used. Set to zero.

dwCallParamFlags		Can be SECURE or ORIGOFFHOOK.

dwCallStates			Call state events we support.

LINEDIALPARAMS    DialParams  Parameters in effect for the call.

dwOrigin			OUTBOUND or EXTERNAL.

dwReason			DIRECT, UNKNOWN or UNAVAIL.

dwCompletionID			Not Used.

dwCountryCode			Zero (0) for Unknown.

dwTrunk			B channel in use for the call. Can be a one or two.

dwCallerIDFlags			BLOCKED, UNAVAIL, or ADDRESS

dwCallerIDSize			Will contain phone number if known.

dwCallerIDOffset

dwCalledIDFlags			Same as above for Caller.

dwCalledIDSize

dwCalledIDOffset

dwDevSpecificSize		Set to device specific options that may be in effect. This will

be an ISCCALLSPECIFIC structure.

dwDevSpecificOffset		Index into buffer of where this record starts.

## Phone Capabilities

The phone device allows the TAPI application  to control some features associated with the analog phone attached to the card. Some items that can be controlled are the ring pattern used for the voice calls, monitoring for digits dialed at the handset and monitor/cause  handset hookswitch changes.

## Data Device

Dialing a Call

To place a data call, the application must do a lineMakeCall. Partial dialing for a data line device is not supported. An application may also supply additional call parameters when making the call. This gives an application access to the Sportster PPP and PPP/ML capabilities among other items.     If this record is not supplied, and the TurboPPP fetaure has not been enabled,  a transparent  connection  is made to another device capable of receiving an ISDN BRI call. Transparent connections are just raw pipes through the card, with  no additional protocol framing performed by the  Sportster Adapter.

Securing A Call

A data call may be secured, using lineSecureCall(), so that a voice call will not be allowed to drop one of its B channels..

Receiving A Call

An application is not required to do an accept of the incoming data call.  This is automatically done by the Sportster if the application does a lineAnswer(). After doing a lineAnswer() the call will transition to the CONNECTED state.

#### lineMakeCall

When making a data call, an application can specify the type of encapsulation it wishes to use by filling out the ISCCALLSPECIFICDATA structure and specifying it in the dwDevSpecificOffset field of the LINECALLPARAMS structure. The default encapsulation mode is Transparent which means none.

## The Voice Device

The  Voice device currently supports POTS level functionality as defined by Technical Note 1: Telephony Functional Level Guidelines. This document is included in Appendix A.

Receiving A Call

An application is not required, nor can it, do an accept of the incoming voice call. This is automatically done by the Sportster. New calls can be generated in one of two ways: There is an incoming voice call or the user lifts the handset. In either case, an offering call state is generated. To determine if the call is at the handset the application can wait for the dial tone event or check the hook switch state (or event) or look at the origin of the call.

Dialing A Call

Voice calls support lineMakeCall and lineDial. A unique property of the Sportster Adapter  is that the handset need not be physically on-hook to dial a call. This condition exists since the handset is internally cross-connected on the board.

## Events Generated

The following events may be generated as qualified by the lines device capabilities.

| CALL STATE EVENTS | DESCRIPTION |
| --- | --- |
| IDLE | Call is not active. |
| OFFERING | Incoming call detected. |
| ACCEPTED | Incoming call setup accepted. |
| DIALTONE | Dialtone is being applied to the line. Modes returned are Normal or Special. |
| DIALING | A number is being dialed. |
| RINGBACK | Destination is being alerted. Ie their phone is ringing. |
| BUSY | Destination number is busy. Modes returned are Busy or Trunk. |
| SPECIALINFO | Special information for a network signal has been received. Only mode Unavailable returned. |
| CONNECTED | Call is connected. |
| PROCEEDING | Call is proceeding though the network. |
| DISCONNECTED | Call has been disconnected. Modes returned are Busy, No Answer, Reject, Normal, Incompatible, Congestion, Unreachable, Bad Address or Unavailable. |

| CALL INFO EVENTS | DESCRIPTION |
| --- | --- |
| RATE | The rate of the call has changed. |
| CALLERID | For an incoming call, means that there is a phone number. |
| CALLEDID | For an outbound call, means that we have a number to dial. |
| ORIGIN | The origin of the call is known. Origin can be External, Internal or Unknown. We are unable to detect PBX calls. |
| OTHER | A data call has been secured. |
| APPSPECIFIC | Application has stored app specific data for the call. |
| MEDIAMODE | Media mode for the call is known. |
| DEVSPECIFIC | Device Specific options are in effect. Can get from the CALLINFO structure. |
| TRUNK | Sent when B channel is negotiated for the call. Set to the value of the B channel selected. |

| DEVICE STATE EVENTS | DESCRIPTION |
| --- | --- |
| RINGING | Sent when an incoming call is being offered. Subsequent ringing events are sent every four seconds until call is answered. |
| REINIT | The SecureLink driver is initializing and attempting to connect to the ISDN switch. |
| CONNECTED | Line is connected to the switch. Level 2 is up and running. |
| DISCONNECTED | Line is disconnected to the switch. Level 2 is not up. |
| OPEN | Line device opened. |
| CLOSE | Line device closed. |
| NUMCALLS | Sent when the number of active calls has changed. |
| INSERVICE | Line is inservice with the switch |
| OUTOFSERVICE | Line is out of service with the switch. |

| ADDRESS STATE EVENTS | DESCRIPTION |
| --- | --- |
| INUSEONE | There is one active call. |
| INUSEZERO | There are no active call.s |
| INUSEMANY | More than one active call on the card. |
| NUMCALLS | The number of active calls has changed. |

| PHONE STATE EVENTS | DESCRIPTION |
| --- | --- |
| RINGMODE | Ring mode for phone has changed. Eight ring modes besides silent are supported. Ringing is done on the card. |
| HANDSET HOOKSWITCH | Handset is either off or on hook. |

| MISCELLANEOUS EVENTS | DESCRIPTION |
| --- | --- |
| PHONE BUTTON | Button pressed on the phone. Up/Down states are simulated. |

| Sportster DEVICE SPECIFIC EVENTS | DESCRIPTION |
| --- | --- |
|  | See the section on Device Specific Extensions for these. |

## Device Specific Extensions

The lineDevSpecific() function call is used as a general extension mechanism in TAPI that enables the Sportster  to allow access to features not offered by other TAPI functions. The meaning of the extensions are device specific to the Sportster, and taking advantage of these extensions requires the application to be fully aware of them.

This operation is part of the Extended Telephony services. It provides access to a device-specific feature without defining its meaning. This operation is only available if the application has successfully negotiated a device-specific extension version.

The extensions offered are described below.

NOTE: It is important to note that the application should lock the memory it has passed within this command and should not re-use it until the call has returned through the LINE_REPLY.

/******************************************************************************

*

*		Copyright U.S. Robotics, 1995, all rights reserved.

*

*  PROGRAM: 		iscdevex.h

*

*  DESCRIPTION:	Device specific extensions in TAPI for the

*					Sportster ISDN 128K.

*

*	The first part of this file details the command  structure which

*	is passed to the TAPI lineDevSpecific() function call. After that,

*	definitions for various structures are specified. Following the

*	structure definitions are the ISC specific events which can be

*	reported through TAPI.

*

*	Modification Log:

*

*	Date		Authors	Notes/Description of Change

*  06-18-95	JLM		Genesis

*

*  12-14-95 	JLM		Changed extension version to 0x00010002 to cover new

*						features.

*

*						Removed the following which were not implemented in the

*						drivers:

*

*						ISC event ISCEVENT_OUTOFSERVICECAUSE.

*						ISCEVENT_OUTOFSERVICE cause codes:

*							ISCOOS_NOTINITED,

*							ISCOOS_SPIDFALIED,

*							ISCOOS_PROTOCOLERR.

*

*						Changed meaning for loWater and highWater in the

*						ISCCALLSPECIFIC structure.

*****************************************************************************/

#ifndef _iscdevext_h_included

#define _iscdevext_h_included

#ifndef _iscconst_h_included

#include "iscconst.h"

#endif

/*****

*  Ensure all programmers are using same byte-alignment for structs.

*****/

#pragma pack(4)

#ifndef TAPIMAXDESTADDRESSSIZE

#define TAPIMAXDESTADDRESSSIZE	80L

#endif

#ifndef TAPIMAXDEVICECLASSSIZE

#define TAPIMAXDEVICECLASSSIZE	40L

#endif

// ***********************************************************************

// This is the extended version number which should be used to

// negotiate the features described in this header file.

// ***********************************************************************

#define ISC_EXT_VERSION		0x00010002

// ***********************************************************************

// This is the device specific extension structure which is passed

// as the argument to lpParams in the lineDevSpecific() call. This is

// the function that is used to enable ISC specific features. The other

// arguments to lineDevSpecific() have meaning depending on the command

// issued.

// ***********************************************************************

typedef struct tagISCDEVSPECIFIC

{

DWORD	dwCommand;	// Device Specific Feature to activate.

LPVOID  lpvArgs;	// Feature specific arguments or NULL.

} ISCDEVSPECIFIC, FAR *LPISCDEVSPECIFIC;

// ***********************************************************************

// The following are the individual commands that can be sent in uCommand

// of the ISCDEBSPECIFIC structure and their related arguments.

// ***********************************************************************

#define	ISC_APP_COMMAND_BASE	100

/*

// Reset board drops all current calls and will reset level one and two

//  and then re-aquire with the switch. This command WILL effect all

//  open line devices, not just the one referenced. It most useful

//  when you have a received a LINEDEVSTATE_DISCONNECTED or OUTOFSERVICE

//  message which could have been caused by a bad board setting, SPID,

//  and so forth.

//

//  lineDevSpecific

//			(

//			hLine 		- Set to line device for the board to reset.

//          dwAddressId	- n/a.

//			hCall		- n/a.

//			lpParams 	- LPISCDEVSPECIFIC.

//			dwSize		- sizeof (ISCDEVSPECIFIC).

//			)

//

//  dwCommand	- ISC_BOARD_RESET.

//  lpvArgs  	- Points to a valid ISCBRICONFIG structure.

*/

#define	ISCCMD_BOARD_RESET			(ISC_APP_COMMAND_BASE + 0)

/*

// This command retrieves statistics for the call referenced.

//

//  lineDevSpecific

//			(

//			hLine 		- Set to line device for the call referenced.

//          dwAddressId	- n/a.

//			hCall		- Call to get stats for. Must be connected.

//			lpParams 	- LPISCDEVSPECIFIC.

//			dwSize		- sizeof (ISCDEVSPECIFIC).

//			)

//

//  dwCommand	- ISC_GET_STATS.

//  lpvArgs  	- Points to a ISCSTATSDATA structure.

*/

#define	ISCCMD_GET_STATS			(ISC_APP_COMMAND_BASE + 1)

/*

// Get board configuration returns the current board configuration.

//

//  lineDevSpecific

//			(

//			hLine 		- Set to line device for the board config

//						  you want.

//          dwAddressId	- n/a.

//			hCall		- n/a.

//			lpParams 	- LPISCDEVSPECIFIC.

//			dwSize		- sizeof (ISCDEVSPECIFIC).

//			)

//

//  dwCommand	- ISC_GET_BOARD_CONFIGURATION.

//  lpvArgs  	- Points to a block of memory large enough to hold

//				  a ISCBRICONFIG structure. The data will be copied into

//				  this location.

*/

#define	ISCCMD_GET_BOARD_CONFIGURATION		(ISC_APP_COMMAND_BASE + 2)

/*

// Sets the default incoming protocol and line features to use for the

// refernced line when answering a call. The default is TRANSPARENT.

//

//  lineDevSpecific

//			(

//			hLine 		- Set to line device.

//          dwAddressId	- n/a.

//			hCall		- n/a.

//			lpParams 	- LPISCDEVSPECIFIC.

//			dwSize		- sizeof (ISCDEVSPECIFIC).

//			)

//

//  dwCommand	- ISC_SET_CALLDEFAULT.

//  lpvArgs  	- Points to a valid ISCCALLSPECIFIC structure.

//

// The encapsulation parameters can be retrieved on a call basis by doing a

//  lineGetcallInfo() and then retrieving the ISCCALLSPECIFIC structure

//  from the dwDevSpecificOffset. They can be retrieved on a line basis

//  by makeing a call to lineGetDevCaps() and lineGetDevStatus() and

//  retrieving the ISCCALLSPECIFIC structure from the dwDevSpecificOffset.

//

// This command will generate a LINEDEVSTATE_DEVSPECIFIC event.

*/

#define	ISCCMD_SET_CALLDEFAULT	(ISC_APP_COMMAND_BASE + 3)

/*

// Sets the line features to use for makeing and receiving calls on

// this line device.

//

//  lineDevSpecific

//			(

//			hLine 		- Set to any line device.

//          dwAddressId	- n/a.

//			hCall		- n/a.

//			lpParams 	- LPISCDEVSPECIFIC.

//			dwSize		- sizeof (ISCDEVSPECIFIC).

//			)

//

//  dwCommand	- ISCCMD_SET_BOARDFEATURES.

//  lpvArgs  	- Points to a valid DWORD which contains the

//					ISCBOARDFEATURES (constants) to use.

//

//  They can be retrieved on a line basis

//  by makeing a call to lineGetDevCaps() and lineGetDevStatus() and

//  retrieving the ISCCALLSPECIFIC structure from the dwDevSpecificOffset.

//

// This command will generate a LINEDEVSTATE_DEVSPECIFIC event.

*/

#define	ISCCMD_SET_BOARDFEATURES	(ISC_APP_COMMAND_BASE + 4)

/*

//  Stop drops all current calls and will cause the device to disconnect

//  from the switch.

//

//  This command WILL effect all open line devices, not just the

//  one referenced.

//

//  lineDevSpecific

//			(

//			hLine 		- Set to line device for the board to stop.

//          dwAddressId	- n/a.

//			hCall		- n/a.

//			lpParams 	- LPISCDEVSPECIFIC.

//			dwSize		- sizeof (ISCDEVSPECIFIC).

//			)

//

//  dwCommand	- ISC_BOARD_STOP.

//  lpvArgs  	- NULL.

*/

#define	ISCCMD_BOARD_STOP			(ISC_APP_COMMAND_BASE + 5)

/*

// Runs board level diagnostics and returns the status of that

//  operation.

//

//  lineDevSpecific

//			(

//			hLine 		- Set to any line device.

//          dwAddressId	- n/a.

//			hCall		- n/a.

//			lpParams 	- LPISCDEVSPECIFIC.

//			dwSize		- sizeof (ISCDEVSPECIFIC).

//			)

//

//  dwCommand	- ISCCMD_RUNBOARDDIAGS

//  lpvArgs  	- Points to a valid ISCDIAGNOSTIC structure.

//

// This command will generate a LINEDEVSTATE_DEVSPECIFIC event.

*/

#define	ISCCMD_RUNBOARDDIAGS	(ISC_APP_COMMAND_BASE + 6)

/*

// Generate a board ringing pattern.

//

//  lineDevSpecific

//			(

//			hLine 		- Set to any line device.

//          dwAddressId	- n/a.

//			hCall		- n/a.

//			lpParams 	- LPISCDEVSPECIFIC.

//			dwSize		- sizeof (ISCDEVSPECIFIC).

//			)

//

//  dwCommand	- ISCCMD_RINGBOARD

//  lpvArgs  	- Points to a valid ISCDIAGNOSTIC structure.

//

// This command will generate a LINEDEVSTATE_DEVSPECIFIC event.

*/

#define	ISCCMD_RINGBOARD	(ISC_APP_COMMAND_BASE + 7)

/*

// Sets the defaults for TurboPPP if the TurboPPP (ML Encapsulation)

// board feature is set. Use of this structure allows you to tailor

// TurboPPP.

//

//  lineDevSpecific

//			(

//			hLine 		- Set to line device.

//          dwAddressId	- n/a.

//			hCall		- n/a.

//			lpParams 	- LPISCDEVSPECIFIC.

//			dwSize		- sizeof (ISCDEVSPECIFIC).

//			)

//

//  dwCommand	- ISCCMD_SET_TURBOPPP_SPECIFIC

//  lpvArgs  	- Points to a valid ISCCALLSPECIFICDATA structure.

//

// This command will generate a LINEDEVSTATE_DEVSPECIFIC event.

*/

#define	ISCCMD_SET_TURBOPPP_SPECIFIC	(ISC_APP_COMMAND_BASE + 8)

// ***********************************************************************

// The following data structures and constants are used with

// the commands above.

// ***********************************************************************

// ***********************************************************************

//					ISCBOARDFEATURE CONSTANTS

//

//	A 0 in that bit position means a feature is diabled, while a 1

//  indicates it is enabled. These features effect the behavior

//  of all line devices on the card for the handling of calls.

// ***********************************************************************

// If VOICEOVERRIDE is enabled, any voice call will cause a second

//  data call to be dropped so that the call can be made.

#define	ISCBOARDFEATURE_VOICEOVERRIDE	  	0x00000001

// If ACCEPTVOICEASDATA is enabled, any call into the line device

//  with a voice type bearer mode will be treated as a data call

//  instead of an inbound analog phone call. This allows devices

//  to make data calls using a voice bearer capability.

#define	ISCBOARDFEATURE_ACCEPTVOICEASDATA	0x00000002

// If BEEPONDATA is enabled, once a data call is connected on the

//  line device, the driver will beep the speaker.

#define	ISCBOARDFEATURE_BEEPONDATA	 		0x00000004

// If PPPEMULATION is enabled, all data calls will be subject to

//  TurboPPP

#define	ISCBOARDFEATURE_PPPEMULATION		0x00000008

// If MPPCPNEGOTIATE is enabled, and the other side can do

//  multilink we will negotiate the control protocols using

//  a multi link header.

#define	ISCBOARDFEATURE_MPPCPNEGOTIATE		0x00000010

// If INCOMINGONDIALUP is enabled, then all incoming digital

//  data calls will "appear" on the dialup networking line

//  device.

#define	ISCBOARDFEATURE_INCOMINGONDIALUP	0x00000020

// ***********************************************************************

//					ISCLINEFEATURE CONSTANTS

//

//	A 0 in that bit position means a feature is diabled, while a 1

//  indicates it is enabled. These are per call features.

// ***********************************************************************

// If 56RETRY is enabled, any call which fails to connect at 64000kbps

//  will automaticall be re-tried at 56000kbps.

#define	ISCLINEFEATURE_56RETRY			0x00000001

// ***********************************************************************

// This is the ISC DIAGNOSTIC structure.

// ***********************************************************************

typedef struct tagISCDIAGNOSTIC

{

DWORD		dwTotalSize;		// Set to size of this structure

DWORD		dwNeededSize;		// Set by driver if more space needed to fulfill request

DWORD		dwUsedSize;		    // Set by driver to amount of data returned.

DWORD		dwIOBase;			// Current board I/O Address

DWORD		dwIRQ;				// Current board IRQ

DWORD		dwRingPattern;		// Pattern to ring (1-8)

DWORD		dwErrorReturn;		// Result of running diagnostic. See

//  SLF_xxxx constants in iscconst.h

DWORD		dwBoardType;		// Board type (SIB_EXTERNAL_BOARD or SIB_INTERNAL_BOARD)

DWORD		dwParallelPortMode;	// Parallel port type.

DWORD		dwReserved1;		// not used - future expansion.

DWORD		dwReserved2;		// not used - future expansion.

} ISCDIAGNOSTIC, FAR *LPISCDIAGNOSTIC;

// ***********************************************************************

// This is the ISC Call Specific data structure used on a

// lineMakeCall or returned on a lineGetCallInfo. It documents the

// protocol encapsulation in effect or invokes it.

// ***********************************************************************

typedef struct tagISCCALLSPECIFICDATA

{

DWORD	dwTotalSize;		// Set to size of this structure

DWORD	dwNeededSize;		// Set by driver if more space needed to fulfill request

DWORD	dwUsedSize;		    // Set by driver to amount of data returned.

DWORD	dwEncapsulation;	// Protocol encapsulation in use.

//   Use Call Encapsulation constants

UINT	protocols;			// PPP protocols to use. Should be OR'd

// together to get what you want. If a

// location is zero, that protocol is not used.

//   Use PPP Protocol constants

// The IP and IPX addresses are encoded as:

//  ddd.ddd.ddd.ddd where ddd is one byte in the int.

DWORD	ipAddress;			// IP address if known or set to

// zero (0) to have it negotiated.

DWORD	ipxNetAddress;		// IPX Network Address if needed.

DWORD	compression;		// Use Compression constants

// The following constants are used for dynamic bandwidth

// expansion when using our PPP/ML or PPP/ML  EMULATION (TurboPPP).

// If traffic over the link exceeds the percentage specified by

// the highWater for the time period specified by loWater then

// a second link is brought up.

//

DWORD	dwHighWater;		// Specifies percentage of line utilization that

// must occur before next B channel is allocated

// to the connection.  Range = 0 thru 100%;

// (0% means next B channel is always allocated,

// 100% means that NO dynamic allocation will

// take place)

DWORD	dwSamplingHistory;	// Specifies the number of seconds of sampling

// history used to determine current throughput

// rates.  Range: 12 - 600 seconds, default = 30

// (TPSH_LOWER, TPSH_UPPER, and TPSH_DEFAULT defined

// in iscconst.h specify these values)

DWORD	dwUseTurboPPPCompression;	// T = use compression, F = dont

DWORD	dwUnexpandTime;		// ... (to be defined later)

// Password for authentication if using PAP or CHAP.

char	password[MAXSLPASSWDSIZE];

// User name for authentication if using PAP or CHAP.

char	username[MAXSLUSERNAMESIZE];

// NetBIOS name

char	netbiosName[MAXSLNBNAMESIZE];

// Second phone number. Needed when using the PPML_ emulation

//  or encapsulation framing. Specifies the number to dial to

//  bring up the second call. Must be specified, even if it

//  is the same as the first.

char	phoneNumber[TAPIMAXDESTADDRESSSIZE];

// The combination of ISCLINEFEATURE_ constants to use or in

//  use on the call.

DWORD	dwLineFeatures;

// This field receives the current combination of ISCBOARDFEATURE_

//  constants which are in effect on the line.  To set board features,

//  use the ISCCMD_SET_BOARDFEATURES device-specific call.

DWORD	dwBoardFeatures;

DWORD		dwReserved1;		// not used - future expansion.

DWORD		dwReserved2;		// not used - future expansion.

}

ISCCALLSPECIFICDATA, FAR *LPISCCALLSPECIFICDATA;

// ***********************************************************************

// This is the statistics structure where statistics info is copied (on

// a per call basis).  NOTE: The following items are gotten from the

// following structs using the TAPI lineGetCallInfo func:

//

// 	CALL SPECIFIC struct:

//		- call protocol

//	 	- compression algorithm

//

// 	CALL INFO:

//		- speed

// 	 	- direction

// 	 	- phone numbers

// ***********************************************************************

typedef struct tagISCSTATSDATA

{

DWORD	dwTotalSize;		// Set to size of this structure

DWORD	dwNeededSize;		// Set by driver if more space needed to fulfill request

DWORD	dwUsedSize;			// Set by driver to amount of data returned.

DWORD	conn_secs;			// Seconds call connected

DWORD	conn_tx_frames;		// Total TX frames for call

DWORD	conn_tx_bytes;		// Total TX bytes for call

DWORD	conn_rx_frames;		// Total RX frames for call

DWORD	conn_rx_bytes;		// Total RX bytes for call

DWORD board_rx_tiny;		// Total Rx msg too small count

DWORD board_rx_big;		// Total Rx msg too large count

DWORD board_tx_tmo;		// Total Tx timeout count

DWORD board_tx_big;		// Total Tx packet too big count

DWORD board_tx_fail;		// Total Tx failure count

DWORD board_tx_eom;		// Total Tx end of msg count

DWORD	board_tx_urun;		// Total TX underruns for call

DWORD	board_rx_orun;		// Total RX overruns for call

DWORD	board_rx_crc_err;	// Total RX CRC errors for call

DWORD dchn_rx_tiny;		// D Channel -Rx msg too small count

DWORD dchn_rx_big;		// D Channel -Rx msg too large count

DWORD dchn_rx_anomalies;	// D Channel -Unexpected Rx H/W

//    status/states

DWORD dchn_tx_tmo;		// D Channel -Tx timeout count

DWORD dchn_rx_byte_cnt;	// D Channel -Rx byte count

DWORD dchn_tx_byte_cnt;	// D Channel -Tx byte count

DWORD	dwReserved1;		// not used - future expansion.

DWORD	dwReserved2;		// not used - future expansion.

}

ISCSTATSDATA, FAR *LPISCSTATSDATA;

/**********************************************************************

*		Definitions to use with the RESET BOARD command.

***********************************************************************/

/*****

*  ISCCHANCONFIG (channel) config struct

*****/

typedef struct tagISCCHANCONFIG

{

unsigned

char		szPhoneNum[TAPIMAXDESTADDRESSSIZE];	// Phone number for B channel

unsigned

char		szSPID[TAPIMAXDESTADDRESSSIZE];		// SPID for B channel

}

ISCCHANCONFIG, FAR *LPISCCHANCONFIG;

/*****

*  Board configuration struct

*****/

typedef struct tagISCBRICONFIG

{

DWORD	dwTotalSize;		// Set to size of this structure

DWORD	dwNeededSize;		// Set by driver if more space needed to fulfill request

DWORD	dwUsedSize;			// Set by driver to amount of data returned.

DWORD			dwTelcoSwitch;		// (Use Switch Type constants)

DWORD			dwIOBase;			// Board I/O Address

DWORD			dwIRQ;				// Board IRQ

// This shoudl be one or two based upon

// the number of phone numbers in use.

// This controls access to the chan

// configuration. That is, if it is

// a one then only chan[0] is valid.

// If it is a two then chan[0] and chan[1]

// are valid.

DWORD			dwBoardType;		// Board type (SIB_EXTERNAL_BOARD or

//             SIB_INTERNAL_BOARD)

DWORD			dwParallelPortMode;	// Parallel port type (4-bit , EPP, ECP

//   or 8 bit ).

#if !defined(CMPL_LST_VRSN_THT_WRKD)

DWORD			dwPassThruMode;		// How we should handle pass through

//  See PASSTHRUMODE_ constants in

//  iscslink.h.

// device name for parallel port

char			szPortName[MAXPORTNAMESIZE];

#endif

// B Channel info for card

DWORD			dwNumPhoneNumbers;

ISCCHANCONFIG	chan[MAXSLBRICHANS];

// True if board is a U interface,

//  else it is a S/T.

//  This field is read only.

BOOL			fIsUTermination;

// True if a data only board.

//  This field is read only.

BOOL			fDataOnly;

// True if board supports external

// ring generator.

//  This field is read only.

BOOL			fExternalRing;

// Cards serial number burned

// into eeprom. This is NULL

// terminated.

//  This field is read only.

unsigned

char			szSerNumber[MAXSERIALNUMBERSIZE];

// Cards MAC address burned

// into eeprom. This is NOT

// NULL terminated.

//  This field is read only.

unsigned

char 			ucMacAddress[MACADDRESSSIZE];

DWORD	dwReserved1;		// not used - future expansion.

DWORD	dwReserved2;		// not used - future expansion.

}

ISCBRICONFIG, FAR *LPISCBRICONFIG;

// ***********************************************************************

// The following are SecureLink Device Specific Events. These events will

// be reported using the LINE_DEVSPECIFIC TAPI Message Type.

// ***********************************************************************

/*

// This event may sent whenever a call terminates with an UNKNOWN

//  disconnect mode which was caused by receiving a specific ISDN cause

//  code that could not be translated to a TAPI disconnect mode.

//

// hdLine:    Line device event occurred on.

// htCall:    Call that is using compression.

// dwMsg:     LINE_DEVSPECIFIC

// dwParam1:  ISCEVENT_ISDNCAUSECODE

// dwParam2:  ISDN cause code received from the network. See CCITT spec.

// dwParam3:  n/a.

*/

#define	ISCEVENT_ISDNCAUSECODE	  0x00000001

/*

// This message is sent if the peer in PPP does not support

//  compression.

//

// hdLine:    Line device event occurred on.

// htCall:    PPP Call we tried to negotiate compression for.

// dwMsg:     LINE_DEVSPECIFIC

// dwParam1:  ISCEVENT_COMPNOTENABLED

// dwParam2:  n/a

// dwParam3:  n/a

*/

#define	ISCEVENT_COMPNOTENABLED			0x00000002

/*

// This message is sent if the peer in PPP does not support

//   a particulatr protocol and we are disabling it.

//

// hdLine:    Line device event occurred on.

// htCall:    PPP Call we tried to negotiate compression for.

// dwMsg:     LINE_DEVSPECIFIC

// dwParam1:  ISCEVENT_PROTOCOLDISABLED

// dwParam2:  Protocol that was disabled. See PPP_PROT_ constants.

// dwParam3:  n/a

*/

#define	ISCEVENT_PROTOCOLDISABLED		0x00000003

/*

// This event is sent whenever a PPP link has gone down

//  unexpectedly and we have re-started it.

//

// hdLine:    Line device event occurred on.

// htCall:    PPP call.

// dwMsg:     LINE_DEVSPECIFIC

// dwParam1:  ISCEVENT_PPPRESTART

// dwParam2:  n/a

// dwParam3:  n/a

*/

#define	ISCEVENT_PPPRESTART	  		0x00000004

/*

// This message is sent after CHAP has authenticated the user.

//

// hdLine:    Line device event occurred on.

// htCall:    PPP Call.

// dwMsg:     LINE_DEVSPECIFIC

// dwParam1:  ISCEVENT_AUTHENTICATED

// dwParam2:  TRUE (expanded link in a ML environment).

//			  FALSE(first link in  a ML environment or a PPP link).

// dwParam3:

*/

#define	ISCEVENT_AUTHENTICATED	0x00000006

/*

// These messages are sent whenever a PPP protocol is negotiated.

//  For some protocols, if negotiated, the application can do a

//  lineGetCallInfo() to retrieve further information about the call.

//

// hdLine:    Line device event occurred on.

// htCall:    PPP Call.

// dwMsg:     LINE_DEVSPECIFIC

// dwParam1:  ISCEVENT_PROTOCOLxx

// dwParam2:  See below.

// dwParam3:  See below.

//

*/

#define	ISCEVENT_PROTOCOLPAP	0x00000007	// Password Authentication

#define	ISCEVENT_PROTOCOLCPAP	0x00000008	// Challenege Authentication

#define	ISCEVENT_PROTOCOLBR  	0x00000009	// Bridging

// dwParam2:  n/a

// dwParam3:  n/a

#define	ISCEVENT_PROTOCOLIPX  	0x00000010	// IPX

// Application can do a lineGetCallInfo() to retrieve the mac address

// from the ISCCALLSPECIFIC structure.

// dwParam2:  Contains the IPX address as ddd.ddd.ddd.ddd where ddd is one

//            byte in the int.

// dwParam3:  n/a

#define	ISCEVENT_PROTOCOLNBF  	0x00000011	// NetBeui

// Application can do a lineGetCallInfo() to retrieve the netbios name

// from the ISCCALLSPECIFIC structure.

// dwParam2:  n/a

// dwParam3:  n/a

#define	ISCEVENT_PROTOCOLCCP  	0x00000012	// Compression Control

// dwParam2 = Compression in use. See compression constants.

#define	ISCEVENT_PROTOCOLIP  	0x00000013	// IP

// dwParam2:  Contains the IP address as ddd.ddd.ddd.ddd where ddd is one

//            byte in the int. An address of all zeros means that an IP

//			  address was not negotiated.

// dwParam3:  0 - Address was configured, 1 - Address was negotiated.

/*

// This message is sent whenever a PPP connection is terminated

//  for a non-normal reason.

//

// hdLine:    Line device event occurred on.

// htCall:    Call that IP address is negotiated for.

// dwMsg:     LINE_DEVSPECIFIC

// dwParam1:  ISCEVENT_PPPCAUSE

// dwParam2:  Set to an PPPCAUSE_ constant.

// dwParam3:

*/

#define	ISCEVENT_PPPCAUSE	0x00000014

/*

//  ISCEVENT_PPPCAUSE cause codes for dwParam2.

*/

#define  PPPCAUSE_CONNECTTMO			0x00000001	// Connection timed out.

#define  PPPCAUSE_AUTHENTICATETMO		0x00000002	// Authenticater has not

//  replied.

#define  PPPCAUSE_NOPEERAUTHENTICATION	0x00000003	// Peer has not requested

//  authentication.

#define  PPPCAUSE_INVALIDCHAPRESPONSE	0x00000004	// Invalid CHAP response.

#define  PPPCAUSE_CHAPTMO            	0x00000005	// CHAP challenge timeout.

#define  PPPCAUSE_CHAPMISMATCH         	0x00000006	// PPP Auth Failed, CHAP

//  mismatch.

#define  PPPCAUSE_PAPMISMATCH         	0x00000007	// PPP Auth Failed, PAP

//  mismatch.

/*****

*  Revert to the default packing alignment.  NOTE:  For the Borlandc compiler,

*  there is no way to revert back.  Thus, for Borlandc builds, everyone must

*  be using

*****/

#pragma pack()

#endif	// iscdevext_h_included

/********:ts=4*****************************************************************

*

*		Copyright U.S. Robotics. 1995, all rights reserved.

*

*  PROGRAM: 		iscconst.h

*

*  DESCRIPTION:	Common constants used for function args and struct

*					members for interfacing to the Sportster ISDN 128K

*					with the TAPI device extensions in iscdevex.h

*

*	Modification Log:

*

*	Date	Authors		Notes/Description of Change

* 08-11-95	JLM			Genesis

*****************************************************************************/

#ifndef _iscconst_h_included

#define _iscconst_h_included

/*****

*  Size constants

*****/

#define MACADDRESSSIZE		6		// Length of cards NAC address.

#define MAXSERIALNUMBERSIZE	36      // Length of Cards serial number.

#define	MAXSLBRICHANS		2		// Max chans configurable on a BRI Adapter.

#define	MAXSLNBNAMESIZE		33		// Max NetBIOS name size

#define	MAXSLPASSWDSIZE		24		// Max PPP password size

#define	MAXSLUSERNAMESIZE	32		// Max PPP username size

#define MAXPORTNAMESIZE 	12		// Max parallel port device name

/*****

* Constants for dwPassThruMode field.  Specifies how we should handle

* the printing while connected with the switch (when not connected we

* always pass through).

*****/

#define PASSTHRUMODE_QUEUEPRINTS	1	// Do not allow port to be shared. We

// will send ISCEVENTS to inform

// app when there is an attempt to

// steal the port so they can pause

// the printer.

#define PASSTHRUMODE_PASSIVE		2	// Allow a steal and then on a polling

// basis process the ISDN device. This

// mode  carries some risk in that an

// incoming setup may be missed.

#define PASSTHRUMODE_ACTIVE			3	// Do real resource stealing as

// documented (ha ha) for Windows 95

// for a parallel dongle.

/*****

*  Board type constants (used for all dwBoardType members of the structures

*  defined in ISCDEVEX.H).

*****/

#define SIB_EXTERNAL_BOARD	0		// External (parallel port) Sporster ISDN board

#define SIB_INTERNAL_BOARD	1		// Internal Sporster ISDN board

/*****

*  Parallel port mode constants (used for all dwParallelPortMode members of the

*  structures defined in ISCDEVEX.H). These are the various parallel port modes

*  the External Sportster 128K will work with.

*****/

#if defined(CMPL_LST_VRSN_THT_WRKD)

#define SIB_PARALLEL_MODE_4BIT	1

#define SIB_PARALLEL_MODE_8BIT	2

#define SIB_PARALLEL_MODE_EPP	3

#define SIB_PARALLEL_MODE_ECP	4

#else

#define SIB_PARALLEL_MODE_4BIT	0

#define SIB_PARALLEL_MODE_8BIT	1

#define SIB_PARALLEL_MODE_EPP	2

#define SIB_PARALLEL_MODE_ECP	3

#endif

/*****

*  Call Encapsulation constants

*****/

#define	TRANSPARENT_ENCAPSULATION	0  // no encapsulation

#define	PPP_ENCAPSULATION			1  // 1-chan encapsulation

#define	PPPML_ENCAPSULATION			2  // 2-chan encapsulation

#define	PPP_EMULATION				3  // 1-chan emulation

#define	PPPML_EMULATION				4  // 2-chan emulation

/*****

*  Compression constants

*****/

#define COMPRESS_NONE			0		// No Compression

#define COMPRESS_STAC			1		// Stac Compression enabled

#define COMPRESS_ASCEND			2		// Ascend Compression enabled

#define COMPRESS_MICROSOFT		3		// Microsoft Compression enabled

/*****

*  SecureLink Diagnostics fail codes

*****/

#define SLF_Passed				0		// No fail - passed

#define SLF_NoHw				1		// Can't find SLNK2 at given I/O base

#define SLF_BadDlFile			2		// Bad or missning download file

#define SLF_DlFail				3		// Download failed

#define SLF_InvalidIrq			4		// Invalid IRQ read from EEPROM

#define SLF_IrqFail				5		// Failed IRQ test (did not get interrupts)

#define SLF_MemFail				6		// On board FIFO memory test failed

#define SLF_BchnFail			7		// B channel loopback test failed

#define SLF_AccessFail			8 		// Register write/read file

#define SLF_SccFail				9		// SCC Reg wr/rd test fail

#define SLF_ImaskFail			10      // Can't disable board ints

#define SLF_Fail				11		// Some internal failure.

/*****

*  PPP Protocol constants

*****/

#define PPP_PROT_PAP			0x01	// Use Password Authentication Protocol.

#define PPP_PROT_CHAP			0x02	// Use Challenge Authentication Protocol.

#define PPP_PROT_BR				0x04	// Enable bridging.

#define PPP_PROT_IP				0x08	// Enable IP.

#define PPP_PROT_IPX			0x10	// Enable IPX.

#define PPP_PROT_NBF			0x20	// Enable NetBeui.

#define PPP_PROT_CCP			0x40	// Enable Compression Control Protocol.

//	Should set compression field.

/*****

*  Telco Switch constants

*****/

#define TELCO_NI1 				0		// National ISDN

#define TELCO_ATT				1		// AT&T 5ESS Custom Swtch or G3 PBX

#define TELCO_NT				2		// Northern Telecom DMS-100 Custom

#define TELCO_EURO_ISDN			3		// Euro ISDN

#define TELCO_INS64				4		// The INS 64 switch type

/*****

*  Range for Sampling History (ISCBOARDFEATURESEX::dwSamplingHistory) (in seconds)x

*****/

#define TPSH_LOWER			12			// Minimum sampling history time

#define TPSH_UPPER			600			// Maximum sampling history time

#define TPSH_DEFAULT		30			// Default sampling history time

/*****

*  Switch types greater than this are not allowed for project

*****/

#if defined(_EURO_ISDN_SWITCHES)

#define TELCO_MAX_ALLOWED		TELCO_INS64

#else

#define TELCO_MAX_ALLOWED		TELCO_NT

#endif

#endif // _iscconst_h_included

# STREAMS LIBRARY

Data processing (or the receiving and sending of packets)  can be done for an active call after the application has  opened the data stream for the call. Once  the stream is open, the application  can send and receive data packets for that connection. Once the call has been dis-connected, the data stream  is no longer valid and should be closed.

It is important to note that any functions called by an application to send, receive or act on the data stream are done though a set of library calls which encapsulate the calls down to the ISDN Driver.

## IscOpenStream

This function call will initialize the data stream for the call specified. The application must have previously done a lineGetID() to obtain the stream identifier.

Syntax

GCA_STATUS

IscOpenStream

(

IN	HLINE		hLine,

IN	HCALL		hCall,

IN    	UINT		uReceiveSize,

IN	UINT		uTransmitSize,

IN	UINT		uStreamType,

IN	DWORD	dwStreamId,

OUT	LPISCHANDLE	 lphIscHandle

)

Parameters

hLine		- Handle to the open line device.

hCall		- Handle to the call to get the data stream for.

uReceiveSize	- Size of the receive ring buffer to create. 0 means use default.

uTransmitSize     - Size of the transmit ring buffer to create. 0 means use default.

uStreamType	- Type of stream to open (data, voice, etc). This is currently not used

but should be set to data.

dwStreamId	- Stream identifier returned from lineGetID() for hCall.

LPISCHANDLE - Handle to the open data stream returned.

Returns

GCA_STATUS_FAILURE	- The stream could not be opened.

GCA_STATUS_SUCCES	S	- The stream was opened.

## IscCloseStream

This command will close the referenced data stream. To gain access to the stream again to receive and transmit packets, the application will need to perform another stream open.

Syntax

GCA_STATUS

IscCloseStream

(

IN	ISCHANDLE	hIscHandle

)

Parameters

hIscHandle	Handle to the open data stream.

Returns

GCA_STATUS_SUCCESS	- Always okay.

## IscWriteStream

After opening the data stream, an application can then start to transmit packets over the connection using the IscStreamWrite function.

This function request instructs the GCA driver to transmit a packet through the adapter and onto the ISDN network.

Transmit packets may be chained together through the NextPacket member of the GCA packet structure. The last packet in the chain should set this value to NULL.

Interrupts may be in any state during this call.

Syntax

GCA_STATUS

IscWriteStream

(

IN    ISCHANDLE  hIscHandle,

IN   BYTE	FAR *lpPacket,

IN   UINT	uPacketSize,

IN   UINT 	Flags

)

Parameters

hIscHandle	Handle to the open data stream.

lpPacket		Pointer to buffer containing data to transmit.

uSize		Size of the data buffer to  transmit.

Flags		Additional commands for the packet which were not included in the

data packet.

Returns

GCA_STATUS_FAILURE	- There was an unrecoverable error writing the packet.

GCA_STATUS_SUCCESS	- The packet was queued for transmission.

GCA_STATUS_NOT_READY	- Flow control. The driver is currently unable to accpet the

packet.

## IscReadStream

This function will read a packet from the stream.

Syntax

GCA_STATUS

IscReadStream

(

IN	ISCHANDLE	hIscHandle,

IN    	UINT		uFlags,

IN	UINT		uBufferSize,

OUT	BYTE	FAR	*lpPacket,

OUT	UINT		*lpuSize,

)

Parameters

hIscHandle	Handle to the opne data stream.

lpPacket		Pointer to buffer to copy received packet into.

uBufferSize	Size of the buffer pointed to by lpPacket.

lpuSize		Size of the received data.

uFlags		Can specifiy a PEEK, in which case the packet is not removed from the

receive buffer.

Returns

GCA_STATUS_FAILURE	- Some un-recoverable error.

GCA_STATUS_SUCCESS	- Read the packet okay.

GCA_STATUS_NO_PACKET	- There were no packets to read.

GCA_STATUS_TOO_SMALL	- The buffer pointed to lpPacket was too small to hold the

received data.

## IscStreamLookAhead

This function will look into the receive stream to see if there any packets to read.

Syntax

UINT

IscStreamLookAhead

(

IN	ISCHANDLE	hIscHandle

)

Parameters

hIscHandle	Handle to the opne data stream.

Returns

0	- There were no packets to read.

n	- Where n is the size of the packet to be read.

## Invalidating A Data Handle

A data handle is valid as long as the call is active and connected or the data stream has been closed Once the call is no longer connected the handle becomes invalid and should no longer be used. An application needs to close the data stream using the IscCloseStream call.

# WINISDN

The Sportster Adapter has implemented the WinISDN model for ISDN connections to the Internet and ISDN Connections from a Peer. For more information on this, please reference the WinISDN specification which is included on the SDK.

# - Telephony Functional Level Guidelines

Windows® Telephony

Technical Note 1: Telephony Functional Level Guidelines

Abstract

This note outlines a proposed classification of Windows Telephony functions into functional levels. There are groups of functions that logically should be implemented together.  If one function from a group is implemented, it makes sense to implement the others in the group.  This classification should aid developers in matching up applications, service providers, and hardware that meet their specific needs.

Comments on this technical note are invited and encouraged, as it is our intention to amplify, solidify, formalize, and promulgate this classification further in the near future. Implementors of Telephony applications and service providers desiring to comment on this note should send email to “telephon@microsoft.com” or a fax to “Windows Telephony Coordinator” at +1 (206) 936-7329.

Purpose

The TAPI functional levels provide a simplified method by which developers can match up applications, service providers, and hardware that meets their specific needs. The levels definition may affect development priorities, advertising, packaging, customer support, etc., but has no impact on the Telephony API and SPI definitions themselves.

Application and service provider developers benefit by using the functional levels as a set of guidelines about market segments, concentrating on features that better “fill out” a given functional class rather than offering only a few features spread across many categories. If the guidelines are followed by applications developers and service provider writers, then the likelihood increases that both will produce products with a maximum degree of functional intersection.

Constraints

The functional levels are primarily a taxonomy of Windows Telephony functionality. To aid a developer faced with mastering the Telephony API or SPI, the projection of functional classes onto actual API or SPI elements (functions, parameters, messages, data structures) is useful information.

There is no intent to enforce correct product classification. Its usage is entirely voluntary, although should be generally encouraged.  Standard market mechanisms are assumed to take care of enforcement.

Functional Levels

From a real-world viewpoint, Windows Telephony functionality is best segmented into two major areas: (1) a call control area, and (2) a miscellaneous other area - due to the lack of better names. The call control area describes the telephone network/switch environment, while the other area describes additional features that are not directly call control related.  We propose the following call control categories.

Call Control Categories

These are somewhat hierarchically related. A Telephony product would likely be described by the “highest” functional category it supports.

·	Assisted Telephony client application (“Telephony-enabled application”)

These are Windows applications that make calls but don’t actually manage their own calls.  The management of these calls is performed by Assisted Telephony server applications described later.

·	POTS (Plain Old Telephone Service), PSTN (Public Switched Telephone Network), analog telephony, analog line

Provides making calls and answering calls; equivalent to “Basic Telephony” in the specification.

·	POTS+, POTS/CCS (Custom Calling Service)

Provides everything in POTS, plus call waiting, cancel call waiting, call forwarding, three-way calling, busy number redial, distinctive ring. Other CCS features such as speed calling are not really useful in a PC environment.

It is not entirely clear whether CLASS services (caller-ID, call blocking, etc.) belong to this category or belong to the “miscellaneous other” area. To most residential users CCS and CLASS are all just service purchased from the phone company with no further distinction made.

·	PBX/Centrex (Private Branch Exchange)

Provides POTS plus all supplementary call control features; it includes most of the previous ones.

·	ISDN (Integrated Services Digital Network)

Provides ISDN features; although even here users are likely to subscribe to different services.

Miscellaneous Other Categories

These are mostly unrelated and independent; they are only meaningful when augmenting call control categories.  A Telephony product can support a variety of the miscellaneous categories (in addition to its call control category).

·	CLASS, Caller-ID

Provides increased information reporting (caller-ID) and usage of features that rely on caller-ID.

·	Assisted TAPI server application

These are applications, referred to as “call managers” in the TAPI specification, that receive and handle Assisted Telephony requests (tapiRequestMakeCall, tapiRequestMediaCall).

·	DTMF and tones (Dual Tone Multiple Frequency, Touch-tone)

Provides detection and generation of DTMF, pulse and inband tones.

·	Media monitoring

Provides the detection of media modes on the information channel on calls.

·	Media Device Classes

Provides access to a call’s media stream via media stream API(s); voice, fax, data, etc.

·	Media control

Provides the limited manipulation of a call’s media stream based on various conditions.

·	Phone device

Provides control of the phone set (hookswitch, ringer, buttons, lamps, display).

While these categories provide a reasonably clean real-world mapping, it is unreasonable to expect that all possible features in each category would be available. End-user feature availability is defined by the intersection of application features, service provider software and hardware features, and telephone network/switch features actually available and enabled/authorized for the specific user.

For the application or service provider/hardware components to completely describe its functionality, it must list (1) the call control category and as well as the individual call control features it supports, and (2) the miscellaneous other categories as well as the individual other features within those categories it supports.

Example: “POTS+ with Caller-ID, DTMF, Assisted Telephony server, Wave device class”.

If a provider or application makes use of certain device-specific extensions, then these would have to be indicated as another “miscellaneous” category.

Projection onto the Telephony API

Mapping categories onto the Telephony API involves more than just grouping the API functions and messages. For example, the TAPI function for making an outbound call takes several parameters. Depending on the function category, different sets/values for some of these parameters will need to be supported. The callback messages and the data structure fields that are valid and the values they report will differ across categories. Completely describing the mapping for a given category involves providing a list of API functions, parameter ranges for those functions, messages, parameter ranges for those messages, as well as listing data structures with field ranges and value ranges.

Each mapping essentially defines an API subset (a range to be exact). An application writer who wants to write an application for a given functional category (or set of categories) only needs to write to the corresponding API subset, and would probably try to cover as much as possible in the subset.

Category: Assisted TAPI Client

tapiRequestMakeCall

tapiRequestMediaCall

tapiRequestDrop

tapiGetLocationInfo

TAPI_REPLY

Category: Line Device Extensions

lineNegotiateExtVersion

lineDevSpecific

lineDevSpecificFeature

LINE_DEVSPECIFIC

LINE_DEVSPECIFICFEATURE

Category: Phone Device

phoneInitialize

phoneShutdown

PHONE_REPLY

phoneNegotiateAPIVersion

phoneGetDevCaps

phoneOpen

phoneClose

phoneGetStatusMessages

phoneSetStatusMessages

phoneGetStatus

phoneGetID (for phone device)

PHONE_CLOSE

PHONE_STATE

phoneGetButtonInfo

phoneSetButtonInfo

PHONE_BUTTON

phoneGetDisplay

phoneSetDisplay

phoneGetVolume

phoneSetVolume

phoneGetGain

phoneSetGain

phoneGetLamp

phoneSetLamp

phoneGetHookSwitch

phoneSetHookSwitch

phoneGetRing

phoneSetRing

phoneGetData

phoneSetData

Category: Phone Device Extensions

phoneNegotiateExtVersion

phoneDevSpecific

PHONE_DEVSPECIFIC

|  | POTS | POTS+ | PBX | ISDN |  | CLASS | Assisted TAPI | Digits Tones | Media Mon | Media Control |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lineAccept |  |  |  | · |  |  |  |  |  |  |
| lineAddToConference |  |  | · | · |  |  |  |  |  |  |
| lineAnswer | · | · | · | · |  |  |  |  |  |  |
| lineBlindTransfer |  |  | · | · |  |  |  |  |  |  |
| lineClose | · | · | · | · |  |  |  |  |  |  |
| lineCompleteCall |  |  | · | · |  |  |  |  |  |  |
| lineCompleteTransfer |  |  | · | · |  |  |  |  |  |  |
| lineConfigDialog | · | · | · | · |  |  |  |  |  |  |
| lineDeallocateCall | · | · | · | · |  |  |  |  |  |  |
| lineDevSpecific |  |  |  |  |  |  |  |  |  |  |
| lineDevSpecificFeature |  |  |  |  |  |  |  |  |  |  |
| lineDial | · | · | · | · |  |  |  |  |  |  |
| lineDrop | · | · | · | · |  |  |  |  |  |  |
| lineForward |  |  | · | · |  | · |  |  |  |  |
| lineGatherDigits |  |  |  |  |  |  |  | · |  |  |
| lineGenerateDigits |  |  |  |  |  |  |  | · |  |  |
| lineGenerateTone |  |  |  |  |  |  |  | · |  |  |
| lineGetAddressCaps | · | · | · | · |  |  |  |  |  |  |
| lineGetAddressID | · | · | · | · |  |  |  |  |  |  |
| lineGetAddressStatus | · | · | · | · |  |  |  |  |  |  |
| lineGetCallInfo | · | · | · | · |  |  |  |  |  |  |
| lineGetCallStatus | · | · | · | · |  |  |  |  |  |  |
| lineGetConfRelatedCalls |  |  | · | · |  |  |  |  |  |  |
| lineGetDevCaps | · | · | · | · |  |  |  |  |  |  |
| lineGetDevConfig | · | · | · | · |  |  |  |  |  |  |
| lineGetIcon | · | · | · | · |  |  |  |  |  |  |
| lineGetID | · | · | · | · |  |  |  |  |  |  |
| lineGetLineDevStatus | · | · | · | · |  |  |  |  |  |  |
| lineGetNewCalls | · | · | · | · |  |  |  |  |  |  |
| lineGetNumRings | · | · | · | · |  |  |  |  |  |  |
| lineGetRequest |  |  |  |  |  |  | · |  |  |  |
| lineGetStatusMessages | · | · | · | · |  |  |  |  |  |  |
| lineGetTranslateCaps | · | · | · | · |  |  |  |  |  |  |
| lineHandoff | · | · | · | · |  |  |  |  |  |  |
| lineHold |  |  | · | · |  |  |  |  |  |  |
| lineInitialize | · | · | · | · |  |  |  |  |  |  |
| lineMakeCall | · | · | · | · |  |  |  |  |  |  |
| lineMonitorDigits |  |  |  |  |  |  |  | · |  |  |
| lineMonitorMedia |  |  |  |  |  |  |  |  | · |  |
| lineMonitorTones |  |  |  |  |  |  |  | · |  |  |
| lineNegotiateAPIVersion | · | · | · | · |  |  |  |  |  |  |
| lineNegotiateExtVersion |  |  |  |  |  |  |  |  |  |  |
| lineOpen | · | · | · | · |  |  |  |  |  |  |
| linePark |  |  | · | · |  |  |  |  |  |  |
| linePickup |  |  | · | · |  |  |  |  |  |  |
| linePrepareAddToConference |  |  | · | · |  |  |  |  |  |  |
| lineRedirect |  |  | · | · |  |  |  |  |  |  |
| lineRegisterRequestRecipient |  |  |  |  |  |  | · |  |  |  |
| lineRemoveFromConference |  |  | · | · |  |  |  |  |  |  |
| lineSecureCall |  | · |  |  |  |  |  |  |  |  |
| lineSendUserUserInfo |  |  |  | · |  |  |  |  |  |  |
| lineSetAppSpecific | · | · | · | · |  |  |  |  |  |  |
| lineSetCallParams |  |  |  | · |  |  |  |  |  |  |
| lineSetCallPrivilege | · | · | · | · |  |  |  |  |  |  |
| lineSetCurrentLocation | · | · | · | · |  |  |  |  |  |  |
| lineSetDevConfig | · | · | · | · |  |  |  |  |  |  |
| lineSetMediaControl |  |  |  |  |  |  |  |  |  | · |
| lineSetMediaMode |  |  |  |  |  |  |  |  | · |  |
| lineSetNumRings | · | · | · | · |  |  |  |  |  |  |
| lineSetStatusMessages | · | · | · | · |  |  |  |  |  |  |
| lineSetTerminal |  |  |  |  |  |  |  |  |  |  |
| lineSetTollList | · | · | · | · |  |  |  |  |  |  |
| lineSetupConference |  |  | · | · |  |  |  |  |  |  |
| lineSetupTransfer |  |  | · | · |  |  |  |  |  |  |
| lineShutdown | · | · | · | · |  |  |  |  |  |  |
| lineSwapHold |  | · | · | · |  |  |  |  |  |  |
| lineTranslateAddress | · | · | · | · |  |  |  |  |  |  |
| lineUncompleteCall |  |  | · | · |  |  |  |  |  |  |
| lineUnhold |  |  | · | · |  |  |  |  |  |  |
| lineUnpark |  |  | · | · |  |  |  |  |  |  |
| LINE_ADDRESSSTATE | · | · | · | · |  |  |  |  |  |  |
| LINE_CALLINFO | · | · | · | · |  |  |  |  |  |  |
| LINE_CALLSTATE | · | · | · | · |  |  |  |  |  |  |
| LINE_CLOSE | · | · | · | · |  |  |  |  |  |  |
| LINE_DEVSPECIFIC |  |  |  |  |  |  |  |  |  |  |
| LINE_DEVSPECIFICFEATURE |  |  |  |  |  |  |  |  |  |  |
| LINE_GATHERDIGITS |  |  |  |  |  |  |  | · |  |  |
| LINE_GENERATE |  |  |  |  |  |  |  | · |  |  |
| LINE_LINEDEVSTATE | · | · | · | · |  |  |  |  |  |  |
| LINE_MONITORDIGITS |  |  |  |  |  |  |  | · |  |  |
| LINE_MONITORMEDIA |  |  |  |  |  |  |  |  | · |  |
| LINE_MONITORTONE |  |  |  |  |  |  |  | · |  |  |
| LINE_REPLY | · | · | · | · |  |  |  |  |  |  |
| LINE_REQUEST |  |  |  |  |  |  | · |  |  |  |
| LINEADDRCAPFLAGS | · | · | · | · |  |  |  |  |  |  |
| LINEADDRESSCAPS | · | · | · | · |  |  |  |  |  |  |
| LINEADDRESSMODE | · | · | · | · |  |  |  |  |  |  |
| LINEADDRESSSHARING | · | · | · | · |  |  |  |  |  |  |
| LINEADDRESSSTATE | · | · | · | · |  |  |  |  |  |  |
| LINEADDRESSSTATUS | · | · | · | · |  |  |  |  |  |  |
| LINEADDRFEATURE | · | · | · | · |  |  |  |  |  |  |
| LINEANSWERMODE | · | · | · | · |  |  |  |  |  |  |
| LINEBEARERMODE |  |  |  | · |  |  |  |  |  |  |
| LINEBUSYMODE | · | · | · | · |  |  |  |  |  |  |
| LINECALLCOMPLCOND |  |  | · | · |  |  |  |  |  |  |
| LINECALLCOMPLMODE |  |  | · | · |  |  |  |  |  |  |
| LINECALLFEATURE | · | · | · | · |  |  |  |  |  |  |
| LINECALLINFO | · | · | · | · |  |  |  |  |  |  |
| LINECALLINFOSTATE | · | · | · | · |  |  |  |  |  |  |
| LINECALLLIST | · | · | · | · |  |  |  |  |  |  |
| LINECALLORIGIN |  |  | · | · |  |  |  |  |  |  |
| LINECALLPARAMFLAGS | · | · | · | · |  |  |  |  |  |  |
| LINECALLPARAMS | · | · | · | · |  |  |  |  |  |  |
| LINECALLPARTYID |  | · | · | · |  |  |  |  |  |  |
| LINECALLPRIVILEGE | · | · | · | · |  |  |  |  |  |  |
| LINECALLREASON | · | · | · | · |  |  |  |  |  |  |
| LINECALLSELECT | · | · | · | · |  |  |  |  |  |  |
| LINECALLSTATE | · | · | · | · |  |  |  |  |  |  |
| LINECALLSTATUS | · | · | · | · |  |  |  |  |  |  |
| LINEDEVCAPFLAGS | · | · | · | · |  |  |  |  |  |  |
| LINEDEVCAPS | · | · | · | · |  |  |  |  |  |  |
| LINEDEVSTATE | · | · | · | · |  |  |  |  |  |  |
| LINEDEVSTATUS | · | · | · | · |  |  |  |  |  |  |
| LINEDEVSTATUSFLAGS | · | · | · | · |  |  |  |  |  |  |
| LINEDIALPARAMS | · | · | · | · |  |  |  |  |  |  |
| LINEDIALTONEMODE |  | · | · | · |  |  |  |  |  |  |
| LINEDIGITMODE | · | · |  |  |  |  |  | · |  |  |
| LINEDISCONNECTMODE | · | · | · | · |  |  |  |  |  |  |
| LINEERR | · | · | · | · |  |  |  |  |  |  |
| LINEFEATURE | · | · | · | · |  |  |  |  |  |  |
| LINEFORWARD |  | · | · | · |  |  |  |  |  |  |
| LINEFORWARDLIST |  | · | · | · |  |  |  |  |  |  |
| LINEFORWARDMODE |  | · | · | · |  |  |  |  |  |  |
| LINEGATHERTERM |  |  |  |  |  |  |  | · |  |  |
| LINEGENERATETERM |  |  |  |  |  |  |  | · |  |  |
| LINEGENERATETONE |  |  |  |  |  |  |  | · |  |  |
| LINEMEDIACONTROLCALLSTATE |  |  |  |  |  |  |  |  |  | · |
| LINEMEDIACONTROLDIGIT |  |  |  |  |  |  |  |  |  | · |
| LINEMEDIACONTROLMEDIA |  |  |  |  |  |  |  |  |  | · |
| LINEMEDIACONTROLTONE |  |  |  |  |  |  |  |  |  | · |
| LINEMEDIACONTROL |  |  |  |  |  |  |  |  |  | · |
| LINEMEDIAMODE |  |  |  |  |  |  |  |  | · |  |
| LINEMONITORTONE |  |  |  |  |  |  |  | · |  |  |
| LINEPARKMODE |  |  | · | · |  |  |  |  |  |  |
| LINEREMOVEFROMCONF |  |  | · | · |  |  |  |  |  |  |
| LINEREQMAKECALL |  |  |  |  |  |  | · |  |  |  |
| LINEREQMEDIACALL |  |  |  |  |  |  | · |  |  |  |
| LINEREQUESTMODE |  |  |  |  |  |  | · |  |  |  |
| LINEROAMMODE | · | · | · | · |  |  |  |  |  |  |
| LINESPECIALINFO | · | · | · | · |  |  |  |  |  |  |
| LINETERMCAPS |  |  |  |  |  |  |  |  |  |  |
| LINETERMDEV |  |  |  |  |  |  |  |  |  |  |
| LINETERMMODE |  |  |  |  |  |  |  |  |  |  |
| LINETERMSHARING |  |  |  |  |  |  |  |  |  |  |
| LINETONEMODE | · | · | · |  |  |  |  | · |  |  |
| LINETRANSFERMODE |  |  | · | · |  |  |  |  |  |  |

Projection onto the Telephony SPI

The functional levels and “miscellaneous” categories for the SPI are, for the most part, the same as for the API listed above. The additional functions which appear on the SPI are “administrative” functions, which are required for all levels. Some functions which appear on the API do not appear on the SPI.

Category: Line Device Extensions

TSPI_lineNegotiateExtVersion

TSPI_lineDevSpecific

TSPI_lineDevSpecificFeature

LINE_DEVSPECIFIC

LINE_DEVSPECIFICFEATURE

Category: Phone Device

TSPI_phoneNegotiateTSPIVersion

TSPI_phoneGetDevCaps

TSPI_phoneOpen

TSPI_phoneClose

TSPI_phoneConfigDialog

TSPI_phoneGetIcon

TSPI_phoneSetStatusMessages

TSPI_phoneGetStatus

TSPI_phoneGetID (for phone device)

PHONE_CLOSE

PHONE_STATE

TSPI_phoneGetButtonInfo

TSPI_phoneSetButtonInfo

PHONE_BUTTON

TSPI_phoneGetDisplay

TSPI_phoneSetDisplay

TSPI_phoneGetVolume

TSPI_phoneSetVolume

TSPI_phoneGetGain

TSPI_phoneSetGain

TSPI_phoneGetLamp

TSPI_phoneSetLamp

TSPI_phoneGetHookSwitch

TSPI_phoneSetHookSwitch

TSPI_phoneGetRing

TSPI_phoneSetRing

TSPI_phoneGetData

TSPI_phoneSetData

Category: Phone Device Extensions

TSPI_phoneNegotiateExtVersion

TSPI_phoneSelectExtVersion

TSPI_phoneGetExtensionID

TSPI_phoneDevSpecific

PHONE_DEVSPECIFIC

|  | POTS | POTS+ | PBX | ISDN |  | CLASS | Assisted TAPI | Digits Tones | Media Mon | Media Control |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TSPI_providerConfig | · | · | · | · |  |  |  |  |  |  |
| TSPI_providerInit | · | · | · | · |  |  |  |  |  |  |
| TSPI_providerInstall | · | · | · | · |  |  |  |  |  |  |
| TSPI_providerRemove | · | · | · | · |  |  |  |  |  |  |
| TSPI_providerShutdown | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineAccept |  |  |  | · |  |  |  |  |  |  |
| TSPI_lineAddToConference |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineAnswer | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineBlindTransfer |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineClose | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineCloseCall | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineCompleteCall |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineCompleteTransfer |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineConditionalMediaDetection | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineConfigDialog | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineDevSpecific |  |  |  |  |  |  |  |  |  |  |
| TSPI_lineDevSpecificFeature |  |  |  |  |  |  |  |  |  |  |
| TSPI_lineDial | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineDrop | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineForward |  |  | · | · |  | · |  |  |  |  |
| TSPI_lineGatherDigits |  |  |  |  |  |  |  | · |  |  |
| TSPI_lineGenerateDigits |  |  |  |  |  |  |  | · |  |  |
| TSPI_lineGenerateTone |  |  |  |  |  |  |  | · |  |  |
| TSPI_lineGetAddressCaps | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetAddressID | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetAddressStatus | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetCallAddressID | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetCallInfo | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetCallStatus | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetDevCaps | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetDevConfig | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetExtensionID | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetIcon | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetID | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetLineDevStatus | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineGetNumAddressIDs | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineHold |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineMakeCall | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineMonitorDigits |  |  |  |  |  |  |  | · |  |  |
| TSPI_lineMonitorMedia |  |  |  |  |  |  |  |  | · |  |
| TSPI_lineMonitorTones |  |  |  |  |  |  |  | · |  |  |
| TSPI_lineNegotiateExtVersion |  |  |  |  |  |  |  |  |  |  |
| TSPI_lineNegotiateTSPIVersion | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineOpen | · | · | · | · |  |  |  |  |  |  |
| TSPI_linePark |  |  | · | · |  |  |  |  |  |  |
| TSPI_linePickup |  |  | · | · |  |  |  |  |  |  |
| TSPI_linePrepareAddToConference |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineRedirect |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineRemoveFromConference |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineSecureCall |  | · |  |  |  |  |  |  |  |  |
| TSPI_lineSelectExtVersion |  |  |  |  |  |  |  |  |  |  |
| TSPI_lineSendUserUserInfo |  |  |  | · |  |  |  |  |  |  |
| TSPI_lineSetAppSpecific | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineSetCallParams |  |  |  | · |  |  |  |  |  |  |
| TSPI_lineSetDefaultMediaDetection | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineSetDevConfig | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineSetMediaControl |  |  |  |  |  |  |  |  |  | · |
| TSPI_lineSetMediaMode |  |  |  |  |  |  |  |  | · |  |
| TSPI_lineSetStatusMessages | · | · | · | · |  |  |  |  |  |  |
| TSPI_lineSetTerminal |  |  |  |  |  |  |  |  |  |  |
| TSPI_lineSetupConference |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineSetupTransfer |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineSwapHold |  | · | · | · |  |  |  |  |  |  |
| TSPI_lineUncompleteCall |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineUnhold |  |  | · | · |  |  |  |  |  |  |
| TSPI_lineUnpark |  |  | · | · |  |  |  |  |  |  |
| LINE_ADDRESSSTATE | · | · | · | · |  |  |  |  |  |  |
| LINE_CALLDEVSPECIFIC |  |  |  |  |  |  |  |  |  |  |
| LINE_CALLDEVSPECIFICFEATURE |  |  |  |  |  |  |  |  |  |  |
| LINE_CALLINFO | · | · | · | · |  |  |  |  |  |  |
| LINE_CALLSTATE | · | · | · | · |  |  |  |  |  |  |
| LINE_CLOSE | · | · | · | · |  |  |  |  |  |  |
| LINE_DEVSPECIFIC |  |  |  |  |  |  |  |  |  |  |
| LINE_DEVSPECIFICFEATURE |  |  |  |  |  |  |  |  |  |  |
| LINE_GATHERDIGITS |  |  |  |  |  |  |  | · |  |  |
| LINE_GENERATE |  |  |  |  |  |  |  | · |  |  |
| LINE_LINEDEVSTATE | · | · | · | · |  |  |  |  |  |  |
| LINE_MONITORDIGITS |  |  |  |  |  |  |  | · |  |  |
| LINE_MONITORMEDIA |  |  |  |  |  |  |  |  | · |  |
| LINE_MONITORTONE |  |  |  |  |  |  |  | · |  |  |
| LINE_NEWCALL | · | · | · | · |  |  |  |  |  |  |
| LINEADDRCAPFLAGS | · | · | · | · |  |  |  |  |  |  |
| LINEADDRESSCAPS | · | · | · | · |  |  |  |  |  |  |
| LINEADDRESSMODE | · | · | · | · |  |  |  |  |  |  |
| LINEADDRESSSHARING | · | · | · | · |  |  |  |  |  |  |
| LINEADDRESSSTATE | · | · | · | · |  |  |  |  |  |  |
| LINEADDRESSSTATUS | · | · | · | · |  |  |  |  |  |  |
| LINEADDRFEATURE | · | · | · | · |  |  |  |  |  |  |
| LINEANSWERMODE | · | · | · | · |  |  |  |  |  |  |
| LINEBEARERMODE |  |  |  | · |  |  |  |  |  |  |
| LINEBUSYMODE | · | · | · | · |  |  |  |  |  |  |
| LINECALLCOMPLCOND |  |  | · | · |  |  |  |  |  |  |
| LINECALLCOMPLMODE |  |  | · | · |  |  |  |  |  |  |
| LINECALLFEATURE | · | · | · | · |  |  |  |  |  |  |
| LINECALLINFO | · | · | · | · |  |  |  |  |  |  |
| LINECALLINFOSTATE | · | · | · | · |  |  |  |  |  |  |
| LINECALLORIGIN |  |  | · | · |  |  |  |  |  |  |
| LINECALLPARAMFLAGS | · | · | · | · |  |  |  |  |  |  |
| LINECALLPARAMS | · | · | · | · |  |  |  |  |  |  |
| LINECALLPARTYID |  | · | · | · |  |  |  |  |  |  |
| LINECALLREASON | · | · | · | · |  |  |  |  |  |  |
| LINECALLSELECT | · | · | · | · |  |  |  |  |  |  |
| LINECALLSTATE | · | · | · | · |  |  |  |  |  |  |
| LINECALLSTATUS | · | · | · | · |  |  |  |  |  |  |
| LINEDEVCAPFLAGS | · | · | · | · |  |  |  |  |  |  |
| LINEDEVCAPS | · | · | · | · |  |  |  |  |  |  |
| LINEDEVSTATE | · | · | · | · |  |  |  |  |  |  |
| LINEDEVSTATUS | · | · | · | · |  |  |  |  |  |  |
| LINEDEVSTATUSFLAGS | · | · | · | · |  |  |  |  |  |  |
| LINEDIALPARAMS | · | · | · | · |  |  |  |  |  |  |
| LINEDIALTONEMODE |  | · | · | · |  |  |  |  |  |  |
| LINEDIGITMODE | · | · |  |  |  |  |  | · |  |  |
| LINEDISCONNECTMODE | · | · | · | · |  |  |  |  |  |  |
| LINEERR | · | · | · | · |  |  |  |  |  |  |
| LINEEXTENSIONID | · | · | · | · |  |  |  |  |  |  |
| LINEFEATURE | · | · | · | · |  |  |  |  |  |  |
| LINEFORWARD |  | · | · | · |  |  |  |  |  |  |
| LINEFORWARDLIST |  | · | · | · |  |  |  |  |  |  |
| LINEFORWARDMODE |  | · | · | · |  |  |  |  |  |  |
| LINEGATHERTERM |  |  |  |  |  |  |  | · |  |  |
| LINEGENERATETERM |  |  |  |  |  |  |  | · |  |  |
| LINEGENERATETONE |  |  |  |  |  |  |  | · |  |  |
| LINEMEDIACONTROLCALLSTATE |  |  |  |  |  |  |  |  |  | · |
| LINEMEDIACONTROLDIGIT |  |  |  |  |  |  |  |  |  | · |
| LINEMEDIACONTROLMEDIA |  |  |  |  |  |  |  |  |  | · |
| LINEMEDIACONTROLTONE |  |  |  |  |  |  |  |  |  | · |
| LINEMEDIACONTROL |  |  |  |  |  |  |  |  |  | · |
| LINEMEDIAMODE |  |  |  |  |  |  |  |  | · |  |
| LINEMONITORTONE |  |  |  |  |  |  |  | · |  |  |
| LINEPARKMODE |  |  | · | · |  |  |  |  |  |  |
| LINEREMOVEFROMCONF |  |  | · | · |  |  |  |  |  |  |
| LINEROAMMODE | · | · | · | · |  |  |  |  |  |  |
| LINESPECIALINFO | · | · | · | · |  |  |  |  |  |  |
| LINETERMCAPS |  |  |  |  |  |  |  |  |  |  |
| LINETERMDEV |  |  |  |  |  |  |  |  |  |  |
| LINETERMMODE |  |  |  |  |  |  |  |  |  |  |
| LINETERMSHARING |  |  |  |  |  |  |  |  |  |  |
| LINETONEMODE | · | · | · |  |  |  |  | · |  |  |
| LINETRANSFERMODE |  |  | · | · |  |  |  |  |  |  |

Open Issues

Note that it is our intention to expand the tables above to include a finer level of detail in future versions of this note. For example, the categorization of support for each LINEDEVCAPFLAGS and LINEADDRCAPFLAGS bit will be included. Also, depending on feedback received, some adjustments may be made to the assignment of features to levels, some “miscellaneous” categories may be merged or further divided, etc.

Other windows applications work transparently over the dial-up networking driver.

This function takes as arguments those parameters required by TAPI (such as speed, phone number, etc.,.) and those optional parameters, as needed, for specific Sportster  protocol functions.

A phone device is supported only for cards which have an RJ-11 jack.

See the ISCCALLSPECIFIC data strucutre for options that can be used when makeing a call.

An active call here means one that has been connected.